CATEGORY 300

DRAINAGE

Book jobs will have the SPI 308, SP308 (for Book Jobs) and SP308.01.04 (Incentives/Liquidated Damages).SECTION 308 — EROSION AND SEDIMENT

CONTROL

308.01 DESCRIPTION.

253	ADD:  The following after the third paragraph.

General Notes.

(a)	Notification.  If an Erosion and Sediment Control Approval is issued for this project, notify the Regional Environmental Coordinator in writing or by telephone at 410-365-0164 prior to the following milestones:

(1)	Pre-construction meeting.

(2)	Erosion and sediment control meeting (minimum 7 working days prior to commencing earth disturbing activities).

(3)	Installation of initial sediment control measures.

(4)	Installation of major sediment control basins/traps.

(5)	Removal or modification of any sediment control structures.

(6)	Removal of all sediment control devices.

(7)	Final acceptance by the Administration.

(b)	Ingress/Egress Controls.  Protect all points of construction ingress and egress to prevent the deposition of materials on public roads. Immediately remove all materials deposited on public roads.  The flushing of road surfaces is prohibited.

Control all ingress and egress points through the use of a stabilized construction entrance. Submit locations for approval by the Regional Environmental Coordinator.

(c)	Inspection.  Inspect all erosion and sediment control measures daily and after storm events.  Maintain continuously in an effective operating condition.

(d)	Shutdowns and/or Penalties.  Total compliance with the approved erosion and sediment control plan is expected at all times.  In cases where the Contractor is found to be in non-compliance, the Administration may take steps to impose partial or total shutdowns and impose per day penalties for non-compliance.

The Administration may impose a total or partial shutdown if the project may adversely impact the waters of the State.

(e)	Record Keeping.  Make the project's approval letter, approved erosion and sediment control plans, approved change requests, daily log books and test reports available on-site for inspection by duly authorized officials.

(f)	Erosion and Sediment Control Excavation.  Place silt removed from control devices in an approved waste site either on or off the project.  Material stored on-site may be reused once it is dried and if it conforms to the Administration’s requirements for embankment or any unspecified need.

(g)	Utility Work.  Follow these additional best management practices for sediment control for utility construction in areas outside of designed controls:

(1)	Call "Miss Utility" at 1-800-257-7777 48 hours prior to the start of work.

(2)	Place excavated material on the high side of the trench.

(3)	Backfill, compact and stabilize trenches for utility installations at the end of each working day.  When this is not possible, follow (4).

(4)	Place temporary silt fences immediately downstream of any disturbed area intended to remain disturbed for more than one day.

(h)	Sensitive Areas.  No construction activities are allowed within specified sensitive areas of the project without prior notification of the Engineer. Designate a responsible party to monitor all work in these areas to assure that reasonable care is taken in or adjacent to these areas.  Areas considered sensitive are defined as: floodplains, wetlands (tidal, nontidal and associated buffers) critical areas, forested areas, archeological sites, historic sites, parkland, and open water.

(i)	Standard Stabilization Note.  Following initial soil disturbance or redisturbance, complete permanent or temporary stabilization within 3 calendar days as to the surface of all perimeter controls, dikes, swales, ditches, perimeter slopes, and all slopes greater than 3:1; and 7 days as to all other disturbed or graded areas on the project site.

(j)	Site Information (Not for Bidding Purposes).

(1)	Total area of site	     acres

(2)	Area disturbed	     acres

(3)	Area to be roofed or paved	     acres

(4)	Total cut	     cubic yards

(5)	Total fill	     cubic yards

(6)	Off-site waste/borrow area location (if known)

(k)	Incremental Stabilization.  Refer to the current Maryland Standards and Specifications for Soil Erosion and Sediment Control for the incremental stabilization of cuts and fills.

(l)	Disturbed Areas.  Place excavated trench material for any storm drain pipe and underdrain pipe installation on the high side of the trench.  Backfill, compact, and stabilize trenches for any storm drain pipe and underdrain pipe installations at the end of each working day.

Stabilize all other disturbed areas at the end of the working day.  Place silt fence downgrade of any areas that cannot be stabilized at the end of the work day such that all runoff from the disturbed area will be filtered.

(m)	Removal of Controls.  Establish permanent stabilization for all contributory disturbed areas and obtain permission prior to the removal of sediment control measures.

Immediately stabilize any areas disturbed by the removal of sediment control measures.

(n)	Notice of Enforcement.  Sediment and erosion control regulations will be strictly enforced.

Design Certification.  Insert the following:

ENVIRONMENTAL INFORMATION

MDE/ PRD #

DESIGN CERTIFICATION

I HEREBY CERTIFY THAT THIS PLAN HAS BEEN DESIGNED IN ACCORDANCE WITH THE MARYLAND STANDARDS AND SPECIFICATIONS FOR SOIL EROSION AND SEDIMENT CONTROL, THE 2000 MARYLAND STORMWATER DESIGN MANUAL, VOLUMES I & II INCLUDING SUPPLEMENTS, THE ENVIRONMENT ARTICLE SECTIONS 4-101 THROUGH 116 AND SECTIONS 4-201 AND 215, AND THE CODE OF MARYLAND REGULATIONS (COMAR) 26.17.01 AND COMAR 26.17.02 FOR EROSION AND SEDIMENT CONTROL AND STORMWATER MANAGEMENT, RESPECTIVELY.

___________________________________        _________________________________

NAME                                                            SIGNATURE

___________________________________	      _________________________________

MARYLAND REGISTRATION NUMBER                                DATE

P.E., R.L.S. OR R.L.A. (circle)

"PROFESSIONAL CERTIFICATION.  I HEREBY CERTIFY THAT THESE DOCUMENTS WERE PREPARED OR APPROVED BY ME, AND THAT I AM A DULY LICENSED PROFESSIONAL ENGINEER UNDER THE LAWS OF THE STATE OF MARYLAND, LICENSE NO.____________________, EXPIRATION DATE:____________________."