CATEGORY 300

DRAINAGE

SECTION 317 — STORMWATER MANAGEMENT (SWM) FACILITY

AS-BUILT CERTIFICATION

317.01 DESCRIPTION.  Submit a certification package that affirms that stormwater management (SWM) facilities and practices are constructed as specified or are functionally equivalent to the designs in the approved SWM Report, revising the certification package as needed until approved.

317.01.01 SWM As-Built Engineer. The SWM As-Built Engineer (ABE) is responsible for assembling and certifying the SWM certification package. Duties include adequately documenting that the SWM facilities have been constructed as specified, and performing inspections during pertinent construction activities for SWM facilities and practices. The ABE shall be a Professional Engineer (P.E.) registered and licensed in the State of Maryland and who has at least three years of experience in SWM facility design and SWM facility construction. Submit one copy of the ABE’s resume to the Engineer. The resume shall include the following.

- Full name of the ABE, License No. and expiration date.

- Name of employing company or firm.

- Contact information.

- Relevant work experience.

- Proof of valid certification of the Maryland Department of the Environment (MDE) Responsible Personnel for Erosion and Sediment Control training course (formerly “Green Card”). Note: All certifications for the former course MDE Responsible Personnel Training for Erosion and Sediment Control (“Green Card”) expired on December 31, 2016 and are no longer valid.

The ABE shall have the option to use designees, who are under the direct supervision of the ABE, to perform the following duties on behalf of the ABE.

- Documenting that the SWM facilities have been constructed as specified, including writing activity inspection reports, taking photographs, and obtaining copies of material approvals and material test results.

- Performing inspections during pertinent construction activities for SWM facilities and practices, completing the pertinent portions of the SWM facility as-built certification data tables.

When the ABE elects to use designees, submit the names and resumes indicating their experience in the design and inspection of SWM facilities, of those designees authorized by the ABE to represent the ABE to the Engineer. Only authorized designees may represent the ABE for the limited duties specified.

317.01.02 SWM Facility As-Built Certification Package. The SWM facility as-built certification package contains documentation that verifies that that all SWM facilities and practices on the Contract have been constructed as specified or are functionally equivalent to the designs in the approved SWM Report.

The SWM facility as-built certification package shall include the following for each SWM facility in the Contract, presented neatly and legibly, and organized in an easy-to-follow format.

- SWM facility construction inspection reports. The inspection reports shall include the following.

  - The SWM facility identification number (BMP No. or SWM Fac. No.) and type of SWM facility or practice.

  - The date and location of the activity.

  - Photographs, taken during inspections, that clearly show the construction activities as listed on the pertinent SWM facility as-built data tables, with narrative descriptions of what appears in the photographs, the dates the photographs were taken, and the locations.

  - Verification of whether SWM facility as-built construction is as specified, noting any deviations from the Contract Documents and how the deviations have been addressed.

- Photographs of SWM facilities and practices after all landscaping has been installed and established, with narrative descriptions of what appears in the photographs.

- Copies of pertinent material approval forms.

- Copies of pertinent material and installation test reports and results.

- Completed as-built certification data tables.

- Green line as-built surveys of the SWM facilities and practices signed and sealed by a Professional Land Surveyor (PLS) who is registered and licensed in the State of Maryland. The as-built survey data shall be overlaid on the appropriate Contract plan sheet(s) and profile sheets, at the same scale and datum, and are coordinately correct. The as-built survey data shall be green in color, clearly legible and easily distinguishable from the Contract Document information. The SWM facility as-built surveys shall include the following.

  - Contours. One-foot contour intervals or otherwise match the contour intervals shown in the Contract Documents. Contours shall cover the entire footprint of the SWM facility or practice as well as inflow and outflow conveyances when ditches or similar features convey runoff into or out of SWM facilities and practices.

  - Drainage Structures. Includes all drainage structures within the footprint of the SWM facility, including but not limited to inlets, manholes, flow splitters, risers, weirs, end sections, headwalls, and end walls. As-built data shall include but is not limited to top of structure elevations, structure lengths, and structure widths; pipe inverts; pipe sizes, materials, and flow directions; orifice elevations; opening sizes; weir dimensions and elevations; check dam locations and dimensions; grates; and trash racks.

  - Riprap and Aggregate. Includes dimensions of riprap and other areas within the footprint of the SWM facility and practice that show a surface layer of aggregate or riprap, including forebays.

  - Embankment Information. Includes embankment heights, widths, and elevations; clay core locations, dimensions, and elevations; cut-off trench locations, dimensions, and elevations; pertinent filter diaphragm information; and pertinent pipe cradle information. Data that cannot be obtained from a field survey shall be provided by the ABE for inclusion with the SWM facility as-built survey.

  - SWM Facility Maintenance Access Roads.

  - Fences. Includes fence that surrounds the footprint of the SWM facility or practice.

  - SWM Facility Profiles. Includes an overlay of green line as-built data on SWM facility profiles and typical sections including but not limited to check dam spacing, check dam top elevations, check dam dimensions, invert elevations, subdrain sizes, subdrain materials, aggregate and soil thicknesses, material types, clay core dimensions, and cut-off trench dimensions. Data that cannot be obtained from a field survey shall be provided by the ABE for inclusion with the SWM facility as-built survey.

  - Certification. Seal, signature, license number, and date of license expiration of the PLS who completes the SWM facility as-built survey.

- Applicable supporting computations demonstrating that the functionality of the SWM facilities and practices meet the approved designs as presented in the approved SWM Report. This is only necessary when tolerances are not met and shall include but is not limited to water surface elevations, freeboard, storage volumes, depths, and other pertinent SWM functionality data that demonstrates the SWM facility performances meets the approved design.

- A narrative of justification for as-built deviations in SWM facilities and practices. This is only necessary when 317.01.02 (g) applies.

- A copy of Final Acceptance from the Administration’s Landscape Operations Division for the landscaping establishment.

- Seal, signature, license number, and date of license expiration of the ABE.

317.01.03 Information Supplied by the Administration. Upon written request, the Administration will provide CADD files in DGN format and the approved Final SWM Report in PDF format to facilitate completion of the SWM facility as-built certification package. Submit requests to the Engineer.

317.01.04 Submittals and Approval Process. Partial submittals of the SWM facility as-built package may be made as construction of each individual SWM facility and practice is completed. Otherwise, submit the entire SWM facility as-built package within 45 days of completion of construction activities associated with all SWM facilities and practices but not including establishment of the specified landscaping items. The landscaping phase of SWM facilities and practices need not be completed to submit the SWM facility as-built certification package for Structural Acceptance but is required for Final Approval.

Resubmit the SWM facility as-built package with responses to all Administration comments that may be received. Resubmit as many times as necessary, updating the SWM facility as-built package as needed to address all Administration comments, and making any field adjustments as needed to correct deficiencies, until Structural Acceptance is issued. Some SWM facility types require approval from the Maryland Department of the Environment (MDE) in addition to approval from the Administration. Resubmit the SWM facility as-built package with responses to all MDE comments that may be received. The Administration will coordinate reviews and correspondence with MDE.

Concurrent with the Administration review of the SWM facility as-built certification package for Structural Acceptance, ensure establishment of landscaping items continues and ensure the area is permanently stabilized.  Once landscaping is established, ensure the remaining data table information is completed and submit the SWM facility as-built certification package for Final Approval.

Submit the SWM facilities as-built package through the Quality Assurance (QA) Toolkit.

317.02 MATERIALS.  Not applicable.

317.03 CONSTRUCTION. Designate an ABE prior to beginning construction of SWM facilities and practices.

Failure to receive approval for the ABE, failure to submit information about the ABE designees, or failure of the ABE, or the ABE designees, to adequately monitor the specified construction activities will be grounds for replacement of the ABE and reconstruction of all work on SWM facilities and practices that may have already been performed.

Perform all construction activities on SWM facilities and practices only in the presence of the ABE or the ABE designee. Failure to perform work in the presence of the ABE or the ABE designee will be grounds for removal and replacement of the ABE, and reconstruction of all work that may have already been performed.

Prior to beginning or continuing construction activities of SWM facilities and practices, ensure the ABE or the ABE designee is present. If the ABE or ABE designee is not present, suspend work on SWM facilities and practices and do not resume until the ABE or ABE designee is present for the activities.

Whenever the ABE or the ABE designee indicates that SWM facilities and practices under construction do not match the Contract Documents, immediately correct the deficiencies before moving to the next construction activity associated with SWM facilities and practices. If it is not possible to correct deficiencies due to the site conditions or constraints and not due to negligence and inadequate quality of work, cease work on SWM facilities and notify the Engineer.

Upon completion of constructing SWM facilities and practices, perform an as-built survey of the completed facility per 317.01.02 (f). Complete installation and establishment of landscaping items need not be completed to perform the as-built survey of SWM facilities and practices.

Submit the SWM facility as-built certification package per 317.01.04. Update SWM facilities as-built surveys when adjustments are made to address comments that may be received.

317.03.01 ABE Responsibilities. 	Ensure that the ABE performs the following.

- Is present for all activities specified on the SWM facilities as-built certification data tables, performs duties as specified, and records requisite information for the SWM facility as-built certification package. The ABE may elect to use a designee as specified in 317.01.01. Ensure the data is available at the Site and on-demand.

- Prepares written inspection reports for construction activities associated with SWM facilities and practices. The ABE may elect to use a designee as specified in 317.01.01.

- Takes photographs during construction activities of the SWM facilities and practices and of the completed SWM facilities, including photographs with completed landscape planting installation and establishment. The ABE may elect to use a designee as specified in 317.01.01.

- Obtains copies of material approvals for items associated with the SWM facilities and practices. The ABE may elect to use a designee as specified in 317.01.01.

- Obtains copies of compaction test results for SWM facility embankments. The ABE may elect to use a designee as specified in 317.01.01.

- Alerts the Contractor when SWM facilities and practices under construction do not match the Contract Documents. The ABE may elect to use a designee as specified in 317.01.01.

- When necessary, performs all computations that demonstrate SWM facilities and practices function in the manner as presented in the approved Final SWM Report, including with all revisions to the report that may result from Redline Revisions. At a minimum, the parameters examined by the ABE shall include but are not limited to storage volumes, discharge rates, velocities, detention times, water surface elevations, freeboard, and all other information as recommended by the ABE and as requested by the Administration.

- Obtains copies of as-built surveys for the SWM facilities and practices.

- Prepares the SWM facility as-built certification package.

317.03.02 Construction Tolerances. As follows. Values outside of tolerance may require computations per 317.03.01 (g).

- Earthwork. Elevations within 3 in. (0.25 ft) of values specified or as otherwise noted on the pertinent SWM facility as-built data table.

- Embankments, Clay Cores, and Cut-Off Trenches. Elevations not less than the values specified.

- Drainage Structures. Elevations within 1.25 in. (0.10 ft) of values specified.

- Pipe Inverts. Elevations within 1.25 in. (0.10 ft) of values specified.

- Riprap. Dimensions within 3 in. (0.25 ft) of values specified.

- Freeboard. Not less than the values specified.

- Aggregate, Sand, Bioretention Soil Mix (BSM), and Mulch Thicknesses. Not less than values specified.

When construction tolerances cannot be met due to unforeseen site conditions or constraints, ensure that calculations are performed by the ABE per 317.03.01 (g) before proceeding with the next construction activity associated with SWM facilities and practices. If, after performing computations, the ABE determines that the SWM facilities do not meet the functional parameters in the approved Final SWM Report as constructed, reconstruct the SWM facilities to meet the functional parameters. If this is not possible due to the site conditions or constraints and not due to negligence and inadequate quality of work, cease work on SWM facilities and notify the Engineer.

317.04 MEASUREMENT AND PAYMENT. Stormwater Management (SWM) Facility As-Built Certification will not be measured but will be paid for at the Contract lump sum price and incrementally distributed per the payment schedule. The payment will be full compensation for all material, labor, equipment, tools, and incidentals necessary to complete the work. No additional compensation will be considered for addressing comments received on the submitted SWM facilities as-built certification package, revisions to the SWM facility as-built certification package, or any construction activities necessary to address comments that may have been received or necessary to revise the SWM facility as-built certification package.

No adjustment to the payment schedule will be made for partial submittals of the SWM facility as-built certification package.

# 317.04.01 Payment Schedule. Payments will be made for the SWM facility as-built certification package as follows.

| # PAYMENT SCHEDULE |  |
| --- | --- |
| # ACTIVITY | # PERCENTAGE OF PAYMENT |
| # Initial submission of the entire SWM Facility As-Built Certification Package. | # 60 |
| # Structural Acceptance from the Highway Hydraulics Division and the Approving Authority. | # 30 |
| # Final Approval from the Highway Hydraulics Division and the Approving Authority. | # 10 |
| # TOTAL | # 100 |

317.04.02 Forfeiture of Payment. Failure to meet the requirements of the payment schedule will result in forfeiture of that percentage of payment.