Use SP 951.07 in conjunction with this specification.

CATEGORY 500

PAVING

DELETE:  SECTION 557 – SNOWPLOWABLE RAISED PAVEMENT MARKERS in its entirety.

INSERT:  The following.

SECTION 557 — SNOWPLOWABLE RAISED PAVEMENT MARKERS

557.01 DESCRIPTION.  Furnish and install new snowplowable raised pavement markers (SRPM) and replacement components for existing markers as specified and as directed, and in conformance with the Maryland Manual on Uniform Traffic Control Devices. Snowplowable Raised Pavement Markers are durable materials.  Select SRPMs from the Qualified Products List (QPL).

557.02 MATERIALS.

Holders		951.07

Pavement Marker Reflector Lenses			951.07

Epoxy Holder Adhesive				951.07

Lens Adhesive					951.07

557.03 CONSTRUCTION.  Per 549.03.

557.03.01 Quality Control.  Per 549.03.01.

557.03.02 Quality Assurance.  Per 549.03.02.

557.03.03 Quality Control Test Strip.  Per 549.03.09.  Install a minimum of 10 groove cuts spaced as specified to verify the accuracy and ability of the equipment and personnel.  Replace any incorrect groove cuts and any incorrect casting placements within the test strip at no additional cost.

557.03.04 Pavement Marker Reflector Lenses.  Reflector lenses for pavement markers shall be the same color as the adjacent pavement marking except the back side shall be as follows.

- One-Way Applications:  The backside for one-way markers shall be red or blank as specified or as directed.

- Two-Way Applications:  The backside for two-way markers shall be the same color as the adjacent pavement marking.

    - General Installation Requirements.

- Ensure all materials and equipment needed to perform the installation are on site.

- Ensure the road surface and ambient temperatures meet the manufacturer’s recommendations for installation. Do not install markers on wet pavement surfaces.

- Replace any incorrect groove cut, incorrect holder placement, or incorrect lens replacement at no additional cost.  An additional test strip may be required in the event of incorrect installations.

557.03.06 New Installation.  Install SRPMs no later than 2 weeks after the completion of the final surface or as directed.

557.03.06.01 SRPMs with Steel Holders

- Install new steel holders with lenses affixed per D 4383 and the manufacturer’s recommendations.  The installed height shall not exceed ¼ in. above the road surface.  Do not install new steel holders that have damaged holders or lenses.

- Install steel holders free of dirt, dust, oil, grease, rust, moisture, or any substances that may impair adhesion to the pavement.  Remove any material that inhibits retroreflectivity of the reflector lens without damage to the lens surface.  Clean each contaminated casting by sand blasting, wire brushing or other approved procedure to remove all substances prior to installation. Do not use chemicals to remove rust.

- Cut the groove for the holder to the appropriate dimensions to allow 1/8 in. side to side movement.  The leading and trailing edges of the holders shall lie below the pavement surface and the holders shall seat properly.

- Use lenses that are specifically manufactured and approved for use as steel holder reflector lenses.  Do not use lenses that are manufactured exclusively for direct application to pavements as substitutes.

557.03.06.02 Recessed SRPMs with Plastic Polycarbonate Holders

- Install SRPM per D 4383 and manufacturer’s recommendations.

- Install plastic holders that are free of dirt, dust, oil, grease, moisture, gloss or any substances that may impair adhesion to the pavement.  Remove any material that inhibits retroreflectivity of the reflector lens without damage to the lens surface.  Clean holders to remove all substances prior to installation. Do not damage the holder.

- Cut the groove for the holder to the appropriate dimensions to allow ¼ in. side to side movement.  All leveling lugs on the holder shall overlap and contact the pavement and the holders shall seat properly.

- Use lenses that are specifically manufactured and approved for use as plastic holder reflector lenses.  Do not use markers that are manufactured exclusively for direct application to pavements as substitutes.

557.03.06.03 Recessed Direct SRPMs without Holders

- Install direct SRPM lenses per D 4383 and manufacturer’s recommendations.  Use lenses manufactured and approved for use as recessed pavement markers.  Installed height of marker shall be 1/4 in. below the pavement surface.

- Install direct lenses that are free of dirt, dust, oil, grease, moisture, or any substances that may impair adhesion to the pavement.  Remove any material that inhibits retroreflectivity of the reflector lens without damage to the lens surface.

- Cut the groove for the direct one-way recessed marker reflectors lenses to the appropriate dimensions for properly seating the reflector lens per D 4383 and the manufacturer’s recommendations.

- Cut the groove for direct two-way recessed marker reflectors lenses to the appropriate dimensions for properly seating both reflectors per D 4383 and the manufacturer’s recommendations.

- Do not use lenses manufactured for use with holders as substitutes for direct lenses unless recommended by manufacturer.

557.03.07 Holder Replacement

- Steel Holder Replacement. Do not replace with damaged or used steel holders.  Do not reuse existing steel holder cuts.  The location of the replacement cut shall be within 10 percent longitudinally ahead (with the direction of traffic) and with no lateral deviation exceeding 1.5 in.  Permanently patch previously used cuts per Sections 504, 505, and 522.

- Polycarbonate Replacement. Do not replace with damaged or used polycarbonate holders.  Existing grooves may be recut for replacement.  Repair damaged groove cuts per Section 504, 505, and 522.  Locate the replacement groove cut within one longitudinal foot ahead of the damaged cut in the direction of traffic with no lateral deviation exceeding 1.5 in

557.03.08 Direct Replacement.

- Remove and dispose of any damaged reflector lenses and replace with lenses that conform to the original lens requirements unless otherwise specified or as directed.  Install replacement lenses per the manufacturer’s recommendations.

- Existing grooves may be recut to comply with D 4383.

- Repair damaged groove cuts per Section 504, 505, and 522.  Locate the replacement groove cut within one longitudinal foot ahead of the damaged cut (with the direction of traffic) with no lateral deviation exceeding 1.5 in.

557.03.09 Reflector Lens Replacement. Remove from holder and dispose of any damaged reflector lens and replace with a new lens on undamaged steel castings or plastic holders that have missing or damaged lenses.  Install replacement lenses per the manufacturer’s recommendations.

557.03.10 Observation Period.  Replace damaged, non-retroreflective or missing Pavement Marker Reflector Lenses within 180 days after opening to traffic at no additional cost.

557.03.11 Submittals.  Submit product data sheets including instructions for installation, MSDS and a Quality Control Plan for review and approval.

557.04 MEASUREMENT AND PAYMENT.  The payment will be full compensation for all pavement preparation, furnishing and placement of SRPM’s, testing, removal, groove cutting, repair and all materials, labor, equipment, tools and all incidentals necessary to complete the work.

Snowplowable Raised Pavement Markers will be measured and paid for at the Contract unit price per each.  The payment includes furnishing and installing SRPM including the holder, reflector, adhesive and grooving.

- SRPMs with Steel Holders will be measured and paid for as specified.

- Recessed SRPMs with Plastic Polycarbonate Holders will be measured and paid for as specified.

- Recessed Direct SRPMs without Holders will be measured and paid for as specified.

Removal of Existing Holders or lenses in groove cuts will be measured and paid for at the Contract unit price per each, excluding any incorrect installations or repairs of groove cuts

Replacement of Pavement Marker Reflector Lenses will be measured and paid for at the Contract unit price per each.