CATEGORY 500

PAVING

SECTION 504 — Asphalt Pavement

504.02 MATERIALS

DELETE:  The following.

Crack Filler			911.01

INSERT:  The following.

Crack Sealer			911.01

504.03 CONSTRUCTION

DELETE:  504.03.03 in its entirety.

INSERT: The following.

504.03.03 Foundation Preparation.  Construct the foundation as specified prior to paving.  Place crack seal and mastic as specified in Section 510.  Remove excess crack sealer and patch material before paving over existing pavement.  Clean, tack coat, fill with asphalt, and tamp spalls and potholes before paving.  Adjust to grade manholes, valve boxes, inlets, and other construction appurtenances within the area to be paved as directed.