Category 500

Paving

SECTION 557 — Snowplowable Raised Pavement Markers

557.03 CONSTRUCTION

DELETE:  557.03.06 New Installation in its entirety.

INSERT:  The following.

557.03.06 New Installation.  Install SRPMs no later than two weeks after the completion of the final surface or as directed.

557.03.06.01 Recessed SRPMs with Plastic Polycarbonate Holders

- Install SRPM according to D4383 and manufacturer’s recommendations.

- Install plastic holders that are free of dirt, dust, oil, grease, moisture, gloss or any substances that may impair adhesion to the pavement.  Remove any material that inhibits retroreflectivity of the reflector lens without damage to the lens surface.  Clean holders to remove all substances prior to installation.  Do not damage the holder.

- Cut the groove for the holder to the appropriate dimensions to allow 1/4 in. side to side movement.  All leveling lugs on the holder shall overlap and contact the pavement and the holders shall seat properly.

- Use lenses that are specifically manufactured and approved for use as plastic holder reflector lenses.  Do not use markers that are manufactured exclusively for direct application to pavements as substitutes.

557.03.06.02 Recessed Direct SRPMs without Holders

- Install direct SRPM lenses according to D4383 and manufacturer’s recommendations.  Use lenses manufactured and approved for use as recessed pavement markers.  Installed height of marker shall be 1/4 in. below the pavement surface.

- Install direct lenses that are free of dirt, dust, oil, grease, moisture, or any substances that may impair adhesion to the pavement.  Remove any material that inhibits retroreflectivity of the reflector lens without damage to the lens surface.

- Cut the groove for the direct one-way recessed marker reflectors lenses to the appropriate dimensions for properly seating the reflector lens according to D4383 and the manufacturer’s recommendations.

- Cut the groove for direct two-way recessed marker reflectors lenses to the appropriate dimensions for properly seating both reflectors according to D4383 and the manufacturer’s recommendations.

- Do not use lenses manufactured for use with holders as substitutes for direct lenses unless recommended by manufacturer.

557.03.07 Holder Replacement

DELETE:  In its entirety.

INSERT:  The following.

557.03.07 Holder Replacement

- Polycarbonate Replacement. Do not replace with damaged or used polycarbonate holders.  Existing grooves may be recut for replacement.  Repair damaged groove cuts as specified in Sections 504, 505, and 522.  Locate the replacement groove cut within one longitudinal ft ahead of the damaged cut in the direction of traffic with no lateral deviation exceeding 1.5 in.

557.04 MEASUREMENT AND PAYMENT

DELETE:  Paragraphs (a), (b), and (c).

INSERT:  The following after the second paragraph, “Snowplowable…adhesive and grooving.

- Recessed SRPMs with Plastic Polycarbonate Holders will be measured and paid for as specified.

- Recessed Direct SRPMs without Holders will be measured and paid for as specified.