Category 400

Structures

SECTION 403 — Temporary Support of Excavation or Fill

- 403.01 DESCRIPTION

Provide temporary support of excavation or fill, as provided in the Contract Documents, for staged construction and as needed for safety and for the support of adjacent pavements and existing structures.

403.02 MATERIALS

Not Applicable.

403.03 CONSTRUCTION

Sheet, shore, brace, and support all excavation or fill contiguous to existing or proposed pavements and existing structures to prevent settlement, movement, or damage to the pavement or structure.  See Section 402 for support requirements for Class 3 Excavation and Section 405 for support requirements for the Removal of Existing Structures.

Design all temporary supports, retained earth structures, sheeting, and shoring by a professional engineer registered in the State of Maryland.  When material retained by the temporary support of excavation or fill is greater than 6 ft high, submit details, procedures, and computations the same as specified for falsework details in TC-4.01 and Section 499.  The experience specified under TC-4.01 will be waived.

Remove temporary support of excavation or fill when no longer needed.  Remove supports in their entirety or as specified in Section 405.

403.04 MEASUREMENT AND PAYMENT

The payment will be full compensation for all excavation, backfilling, materials, shipping, installation, complete or partial removal of temporary support of excavation or fill, and for all material, labor, equipment, tools, and incidentals necessary to complete the work.

403.04.01 Support of Excavation or Fill will not be measured but will be paid for at the Contract lump sum price.

403.04.02 Support of Excavation or Fill will be measured and paid for at the Contract unit price per linear foot, for the actual installed quantity measured.

403.04.03 Support of Excavation or Fill will be measured and paid for at the Contract unit price per square foot as measured vertically from the ground line on the front face (lower face) of the support to the groundline on the back face of the support and measured horizontally along the front face of the temporary support structure.