Category 800

Traffic

SECTION 806 — Luminaires and Lamps

806.03 CONSTRUCTION

806.03.05 Luminaire Photometric Data and Calculations.

DELETE:	 The first paragraph of “(b) Photometric Calculations…for the calculations.”

INSERT: The following.

(b) Photometric Calculations. Printouts of the predicted lighting levels for the areas indicated in the Contract Documents. If no areas are indicated, select a typical section of the project for each type of luminaire. Assume a maintenance factor of 0.81 for the calculations.

DELETE:	 The seventh paragraph of (b) Photometric Calculations “For Light Emitting Diode…greater than 4 to 1.”

INSERT:  The following.

For Light Emitting Diode (LED) Roadway Luminaires, correction factors shall be applied for the lumen retention at 50 000 hours.  The illuminance shall not decrease by more than 10 percent at 50 000 hours, which results in a Lamp Lumen Depreciation (LLD) factor of 0.9.  Apply an additional factor of 0.9 for Luminaire Dirt Depreciation (LDD), to obtain a total maintenance factor of 0.81 for calculations.  Provide a luminaire mounting height of 40 ft with light centers directly over the edge line of the roadway.  Assume four poles in a straight line, parallel to the roadway, spaced at 180 ft each.  Perform calculations for illuminance and luminance based on a R3 class pavement.  The calculation grid shall be based on a two lane road with 12 ft lanes and shall be placed between the center two poles.  Calculate two lines of points for each lane.  The first and the second line of calculation points shall be 4 ft from the left and 4 ft from the right lane lines, respectively.  Start each line of calculation points directly under the second luminaire and continue every 20 ft until directly under the third luminaire.  Each line shall have 10 points, and a total of 40 points shall be calculated.  To be acceptable, the average maintained illuminance of all 40 points shall be 0.9 ft candles or greater with an average to minimum uniformity ratio no greater than 4 to 1.