CATEGORY 900

MATERIALS

SECTION 950 — Traffic Materials

950.12 LUMINAIRES AND LAMPS

950.12.01 Luminaire Construction.

DELETE:	 The first paragraph of (d).

INSERT: The following.

(d) Design LED bracket arm mounted Luminaires for an operational life of at least eleven years with 90 percent lumen maintenance value of 50 000 hours (L90) at an average operating time of 12 hours per night.  The illuminance shall not decrease by more than 10 percent over the minimum operational life of eleven years.  All components of the LED Roadway Luminaires must be rated for the full service life without maintenance.