Category 100

Preliminary

SECTION 104.07 — Arrow Panel (AP)

DELETE: 104.07.03 CONSTRUCTION in its entirety.

INSERT: The following.

104.07.03 CONSTRUCTION

At least 24 hours before use, furnish and test the arrow panel (AP).  The AP shall be self-contained, vehicle-mounted or portable.  Use a self-contained trailer unit unless otherwise specified.

If a full matrix display board is used as an AP, it shall comply to this specification and it shall be able to display a left arrow, right arrow, double arrow, and a four-corner caution mode per the Arrow Panel tables.

Elements.  The AP unit shall conform to the Arrow Panel tables and be arranged with double-pointed arrow configuration capable of displaying a left arrow, right arrow, double arrow, and a four-corner caution mode.  A sequential chevron, sequential arrow and single bar caution panel shall not be displayed.

| ARROW PANEL |  |
| --- | --- |
| UNIT | MdMUTCD TYPE |
| Portable | A, D |
| Vehicle Mounted | B |
| Self-Contained Trailer | C |

| Arrow Board Type | Minimum Size | Minimum Legibility Distance | Minimum Number of Elements |
| --- | --- | --- | --- |
| A | 48 in. x 24 in. | 1/2 mile | 12 |
| B | 60 in. x 30 in. | 3/4 mile | 13 |
| C | 96 in. x 48 in. | 1 mile | 15 |
| D | None* | 1/2 mile | 12 |

∗Length of arrow equals 48 in., width of arrowhead equals 24 in.

Type A, B, and C arrow boards shall have solid rectangular appearances.  A Type D arrow board shall conform to the shape of the arrow.

The color presented by the AP lights shall be yellow.  Lights with the pre-flash option of white strobe or pulsed lighting shall not be displayed.

APs shall have both manual and automatic dimmer devices capable of reducing the light intensity by 50 percent.  Periodically clean the photocells in order to prevent malfunctioning of the brightness control.  Dimmer devices are mandatory during night operation.  The devices shall include a fail-safe system that ensures maximum brightness during daytime operations and a reduction in brightness of up to 50 percent during periods of darkness, regardless of which dimmer device is operational.

The APs shall provide full illumination within at least a 24-degree cone perpendicular to the panel face.

The flashing rate shall be not less than 25 or more than 40 flashes per minute as required in the MdMUTCD.

Power Supply.  The AP shall operate from a solar powered electrical system and consist of battery power and solar array panels and be capable of providing power supply to the AP for 21 consecutive days without auxiliary charge.

Deployment.  Place APs as specified. Maintain APs in good operating order.

- The AP shall conform to the applicable requirements of the MdMUTCD and only be used to supplement other required traffic control devices.  Use the "Arrow" mode when closing a through travel lane on a multilane roadway.  Place only one AP in the "Arrow" mode for each stationary lane closure.  “Arrow” mode shall not be used to indicate a lane shift. Moving work operations may utilize one or more APs for a single lane closure.  Ensure that placement does not cause driver confusion near ramps, median crossovers, and side road intersections.

- Place the AP facing approaching traffic in conformance with the minimum legibility distances specified in the Arrow Panel table.  Ensure that the display is level.

- For stationary lane closures, place the AP on the shoulder at the beginning of the taper (nearest to oncoming traffic).  Where there are narrow or no existing shoulders adjacent to the closed lane behind the channelizing devices, place the AP as near to the beginning of the taper as possible.

- For interior lane closure when the AP is displaying a flashing double arrow, center the AP in the closed lane and place it at the downstream end of the shifting taper.

- For a lane closure on a two-lane, two-way roadway, or for a shoulder closure on roadways, use the “Caution” mode.  In "Caution" mode, one light is displayed in each corner.

(f) For moving operations, refer to Section 104.23.