CATEGORY 100

PRELIMINARY

# SECTION 104.32 — Automated Speed Enforcement (ASE)

104.32.01 DESCRIPTION

Coordinate with the Administration and its Automated Speed Enforcement Contractor for Automated Speed Enforcement (ASE) in the work zone.  Provide work zone signage, speed display trailer, and protection devices to accommodate ASE in the work zone.  Contact the Office of Traffic and Safety Traffic Development and Support Division (OOTS TDSD) ASE program representative prior to fabricating and installing ASE signs to determine exact locations for the ASE vehicle, ASE signs, and speed display trailer.

104.32.02 MATERIALS

| Temporary Concrete Traffic Barrier (TCB) for Maintenance of Traffic | 104.04.02 |
| --- | --- |
| Temporary Traffic Signs (TTS) | 104.08.02 |
| Temporary Traffic Barrier End Treatments | 104.09.02 |
| Temporary Crash Cushion Sand Filled Plastic Barrels (SFPB) | 104.10.02 |
| Drums for Maintenance of Traffic | 104.12.02 |
| Cones for Maintenance of Traffic | 104.14.02 |
| Protection Vehicle (PV) | Section 104.23 |
| Speed Display Trailer (SDT) | 104.28.02 |
| Aggregate for Maintenance of Traffic | 105.02 |
| Reflectorization | 950.03 |

    - CONSTRUCTION

104.32.03.01 General.  Refer to Standard No. MD 104.06-26. Automated Speed Enforcement (ASE) will be used in the work zone to enhance safety for the workers and traveling public.  The Administration will provide a mobile ASE system, including speed enforcement equipment, ASE vehicle, and labor for ASE system operation.  The Contractor shall provide (furnish, install, relocate, and remove) all of the applicable ASE signing, the speed display trailer, and protection devices as required.

Coordinate and cooperate with the Project Engineer and the ASE program representative to implement the ASE system on this project throughout construction.  Obtain approval from the ASE program representative for the ASE vehicle location, ASE sign locations and sizes, and speed display trailer location.

Prior to the start of the project, arrange a field meeting with the ASE program representative to verify the sign types, sizes, and locations.  At this meeting, jointly identify locations for the ASE vehicle and speed display trailer with the ASE program representative.

Contact the ASE program representative a minimum of two weeks prior to all phase changes or construction activities that require relocation of the ASE vehicle.  When activities occur that require relocation of the ASE vehicle, arrange a field meeting with the ASE program representative to verify the suitability of existing and/or identify new location for the ASE vehicle, advance warning signs, speed display trailer, and all required protection devices.

Where feasible, provide a bathroom facility in a location that can be used by construction personnel and the ASE vehicle operators.  Make the bathroom facility available and accessible to the ASE vehicle operators twenty-four hours a day, seven days a week.  Provide the bathroom facility at a location within a reasonable walking distance of the ASE vehicle deployment location and where the ASE vehicle operator does not have to cross travel lanes to access the facility.

104.32.03.02 Temporary Traffic Signs for ASE.  Refer to 104.08.03.  Provide automated speed enforcement advance warning signs on each approach to the work zone, including ramps.  These signs shall consist of ASE advance notice signs and ASE speed limit signs, as shown on the plans and/or as directed by the ASE program representative.  Additional ASE signs may be requested by the ASE program representative based on project needs.  Prior to sign acquisition, coordinate with the ASE program representative to verify the sign types, colors of sign background and legend, materials, sizes, and locations of signs. Relocate and/or install new ASE signs as needed throughout construction.

104.32.03.03 Aggregate Base.  Refer to 105.03.  Where the ASE vehicle deployment location is not on hard surface, construct a level stone entrance and pad adjacent to the roadway for the ASE enforcement vehicle.  Provide a safe means for the ASE vehicle to enter/exit the enforcement area.

104.32.03.04 Temporary Traffic Barrier.  Refer to 104.04.03.  Provide protection for the ASE vehicle, either behind temporary concrete barrier or existing w-beam.  If protecting the ASE vehicle with concrete barrier or existing W-beam traffic barrier is not feasible, supply a protection vehicle with truck or trailer truck mounted attenuator to protect the ASE vehicle.  Coordinate placement of the protection vehicle with the ASE program representative. Refer to Section 104.23.

104.32.03.05 Speed Display Trailer.  Refer to 104.28.03.  Furnish, install, maintain, relocate and remove a speed display trailer as shown on the plans and/or as directed by the ASE program representative.  Construct a level area adjacent to the shoulder for placement of the speed display trailer as needed.  Periodically inspect the speed display trailer and make adjustments as needed to ensure proper function.  Relocate and/or install new speed display trailer as needed throughout construction.

104.32.03.06 Maintenance of Traffic.  Conduct maintenance of traffic as specified in 104.02.03 where required for installation, relocation, or removal of temporary traffic control devices for ASE activities.

104.32.03.07 Temporary Traffic Barrier End Treatments.  Refer to 104.09.03.

104.32.03.08 Temporary Crash Cushion Sand Filled Plastic Barrels.  Refer to 104.10.03.

104.32.03.09 Drums for Maintenance of Traffic.  Refer to 104.12.03.

104.32.03.10 Cones for Maintenance of Traffic.  Refer to 104.14.03.

104.32.04 MEASUREMENT AND PAYMENT

The payment will be full compensation for all material, labor, equipment, tools, and incidentals necessary to complete the work including relocating, turning, completely covering and uncovering or removing and resetting, maintaining in like new condition, and cleaning existing and temporary traffic signs and other traffic control devices related to the operation of the Automated Speed Enforcement.

The pay item will include the following items unless otherwise specified.

104.32.04.01 Maintenance of Traffic for Automated Speed Enforcement will not be measured but will be paid for at the Contract lump sum price.  The payment will be full compensation for relocating, turning, completely covering and uncovering or removing and resetting, maintain in like new condition, and cleaning existing and temporary traffic signs and other traffic control devices related to the operation of the Automated Speed Enforcement Payment of the Contract lump sum price will be prorated and paid in equal amounts on each monthly estimate.  The number of months used for prorating will be the number estimated to complete the work.

104.32.04.02 Temporary Concrete Traffic Barrier and Reset Temporary Concrete Traffic Barrier shall be measured and paid for as specified in 104.04.04.  Reflective Barrier Markers and Vertical Panels shall be measured and paid for as specified in 104.04.04.

104.32.04.03 Temporary Traffic Signs will be measured and paid for as specified in 104.08.04.

When ASE advance warning and speed limit signs are furnished by the Administration or by the Administration’s ASE Contractor, the Contractor will be responsible for set-up, take down and relocation of these signs.  Signs shall be placed per the direction of the ASE program representative and in accordance with the requirements of Section 104.08.  Installation, take down, and relocation of signs provided to the Contractor will not be measured and the cost will be incidental to Maintenance of Traffic for Automated Speed Enforcement.

104.32.04.04 Temporary Traffic Barrier End Treatments, Remove and Reset Temporary Traffic Barrier End Treatments, and Repairing Temporary Traffic Barrier End Treatments will be measured and paid for as specified in 104.09.04.

104.32.04.05 Temporary Crash Cushion Sand Filled Plastic Barrels shall be measured and paid for as specified in 104.10.04.

104.32.04.06 Drums for Maintenance of Traffic shall be measured and paid for as specified in 104.12.04.

104.32.04.07 Cones for Maintenance of Traffic shall be measured and paid for as specified in 104.14.04.

104.32.04.08 If requested, the Protection Vehicle shall be measured and paid for as specified in 104.23.04.

104.32.04.09 Speed Display Trailer will be measured and paid for as specified in 104.28.04.

104.32.04.10 Supply of materials for, installation of, and removal of stone construction entrance(s) and pad(s) for ASE vehicles will be measured and paid for as specified in 105.04.