Category 100

Preliminary

SECTION 111 — Sampling Devices, Testing and Safety Equipment

111.03 CONSTRUCTION

(c) Field Determination … Subbases (MSMT 254).

DELETE:  List “(3) Large spoon or scoop.”

INSERT:  The following.

(3) Dustpan

(f) In-Place Density … Shoulder Material (T 99, T 180, T 191, and MSMT 350).

DELETE:  Lists (7), (8), and (13)

INSERT:  The following:

(7) 12 in. Rocker-type field-testing sieve complete with pan.

(8) One each of the following sieve screens conforming to M 92:

| SIZE (in.) | SHAPE | SIZE OPENINGS |
| --- | --- | --- |
| 12 | Square | 2 in. |
| 12 | Square | 3/4 in. |
| 12 | Square | No. 4 |
| 12 | Square | No. 10 |
| *8 | Round | No. 10 |

*For density sand.

(13) Calibrated sand cone density apparatus.

(j) Other Measuring Devices.

DELETE:  List (1)

INSERT:  The following.

- Handheld pile driving monitoring device as specified in 410.03.02(b) (as approved by the Engineer).

ADD:  The following after list (j).

(k) Field determination of the Volumetric Pavement Macrotexture Depth (MSMT 413).

- Type 1 Glass beads conforming to M 247.

- A Class B or better, style III, 250 mL capacity graduated cylinder per E 1272 used to measure the volume of glass beads for the test.

- A flat, stiff hard disk with a diameter of 2.5 in. to 3.0 in. and a thickness of 1.0 in. is used to spread the glass beads.

NOTE: An ice hockey puck is considered suitable for use as a spreader for this test.

- A stiff wire brush or a soft bristle brush to clean the pavement.

- A small container with a secure and easily removable cover in order to store 200 ml (12.2 in3) of glass beads.

- A shield to protect the test area from air turbulence by the wind or traffic.

(l) Field determination of Pavement Texture Drainage using an Outflow Meter (ASTM E2380).

- Outflow Meter.

- A broom or a handheld wire brush to clean the test area.

- Supply of clean water used to operate the outflow meter.