Category 300

Drainage

Contracts without Project Plans will have the SP 308–No Plans and SP308.04.03 (Incentives/Liquidated Damages).

SECTION 308 — Erosion and Sediment Control

DELETE:  SECTION 308 – EROSION AND SEDIMENT CONTROL in its entirety.

INSERT:  The following.

SECTION 308 — Erosion and Sediment Control

308.01 DESCRIPTION

Install and maintain erosion and sediment control (ESC) measures and best practices throughout the duration of the Contract as specified in the Contract Documents, regulations, regulatory approvals, and regulatory permits.

308.01.01 ESC Approved Plans.  The portion of the Contract Documents consisting of the ESC plans bearing the pertinent regulatory approval from the Administration’s Plan Review Division (PRD) and inclusive of all approved revisions and modifications thereto.

308.01.02 Training, Certifications, and Accounts.  The Erosion and Sediment Control Manager (ESCM) and the Superintendent shall have valid certification from successful completion of the MDE “Responsible Personnel Certification Training for Erosion and Sediment Control” and the Administration’s “Erosion and Sediment Control Certification Training for Contractors and Inspectors.”  The certifications shall be current for the duration of the Contract.

The ESCM and the Superintendent shall maintain active accounts as specified in 308.01.06.

308.01.03 Conformance Requirements.  In addition to the Contract Documents, conform with the latest applicable regulations, regulatory approvals, regulatory permits, and other documents, including but not limited to, the following.

- Annotated Code of Maryland and the Code of Maryland Regulations (COMAR) 26.17.01 (Erosion and Sediment Control), 26.17.02 (Stormwater Management), 26.17.04 (Construction on Nontidal Waters and Floodplains), and 26.17.06 (Water Appropriation or Use).

- Title 9 of the Environment Article, Annotated Code of Maryland and COMAR 26.08.04.

- The provisions of the Federal Clean Water Act (CWA), 33 U.S.C. §1251 et seq., as amended by the Water Quality Act of 1987, and its implementing regulations at 40 Code of Federal Regulations (CFR) Parts 122, 123, 124, 125 and 127.

- 23 CFR §I.G. Part 650 Subpart B.

- 23 U.S.C. 109 Standards (2020) Subsection (g).

- 40 CFR Part 110, 40 CFR Part 117, and 40 CFR Part 302.

- 40 CFR part 112 and Section 311 of the CWA.

- 40 CFR 122.26(a)(1)(v) or 40 CFR 122.26(b)(15)(ii).

- 40 CFR 122.26(b)(14)(x) or 122.26(b)(15)(i).

- 40 CFR 122.44(i)(1)(iv).

- 40 CFR 144 -147.

- General Permit for Stormwater Discharge Associated with Construction Activity General National Pollutant Discharge Elimination System (NPDES) Permit Number MDRC0000 State Discharge Permit Number 20CP0000.

- ESC Handbook, which refers to the handbook codified in COMAR 26.17.01 titled “2011 Maryland Standards and Specifications for Soil Erosion and Sediment Control”, or its successor.

- MDE “2000 Maryland Stormwater Design Manual,” Volumes I and II or its successor.

- SHA “Field Guide for Erosion and Sediment Control.”

- MDE Waterway Construction Guidelines.

308.01.04 Erosion and Sediment Control Manager (ESCM).  The ESCM is responsible for the implementation of the ESC measures and methods of operations, including implementation of corrective actions.  The ESCM shall have the authority to implement ESC, schedules, and methods of operation for both on-site and off-site activities.  When the Contract Documents indicate a limit of disturbance (LOD) is greater than or equal to 1 acre, the ESCM is also a member of the stormwater team. Duties include the following.

- Attend the pre-construction meeting.

- Attend the initial ESC field meeting and periodic field meetings to evaluate the effectiveness of ESC measures installed and to plan for the implementation of additional measures for subsequent areas of soil disturbance, which includes developing a list of activities and schedules to ensure compliance with the Contract Documents and conformance requirements as specified.  Must be on site at a frequency and duration to ensure compliance.

      - Perform daily inspections of the installed ESC measures and ensure they are always in place and functioning.

      - Maintain a daily log of the ESC inspections that includes documenting steps taken for corrective actions, and submit a daily written report to the Engineer at the end of each calendar day.

      - Conduct inspections after a storm event of 0.25 inches or greater, within 24 hours, either the same day the rainfall event concludes or the next day, concurrently with the Engineer, submitting a written report documenting observations and corrective actions, to the Engineer.

      - Authorize mobilization of crews 24 hours a day each calendar day to make immediate repairs and implement corrective actions to ESC measures.  Coordinate with the Engineer and the REC to ensure that all deficiencies are immediately corrected, and that Contract remains in compliance.

      - When requested, accompany the Engineer and Regional Environmental Coordinator (REC) during compliance inspections and inspections made by regulatory agency personnel.

      - Ensure prohibited discharges per 308.01.03(l) are not released and measures are in place to prevent prohibited discharges from occurring.

      - Ensure dewatering activity and turbidity monitoring is performed, recorded, documented, and ensure documentation includes corrective actions taken.

308.01.05 Severe Weather Event.  A weather event in which measured rainfall exceeds 3 in. in a continuous 24-hr period based upon rainfall data obtained from the nearest official National Weather Service (NWS) gauge station in proximity to the specific location of construction activities or from a gauge per 308.03.02 (c).

308.01.06 Quality Assurance (QA) Toolkit.  A web-based system that contains Contract and permit information, ESC inspection reports, stormwater management (SWM) facility as-built certification package submittals and tracking and may be used to submit requests for field modifications to the ESC Approved Plans, as well as track the status thereof, as instructed by the Administration’s “Quality Assurance Toolkit Construction Field Manual.”

(https://apps.roads.maryland.gov/epd.qatoolkit/frmLogin.aspx)

308.01.07 Stormwater Team.  When the Contract Documents indicate an LOD greater than or equal to 1 acre, the Administration will establish a stormwater team.  The stormwater team is not involved with permanent stormwater management (SWM) facilities in any way and is instead a team focused on ESC and pollution prevention during construction activities.

The stormwater team includes personnel who are responsible for the following.

- Compliance with the installation, maintenance, and repair of ESC measures and pollution prevention.

- Conducting inspections as specified.

- Taking corrective actions.

The stormwater team shall have sufficient knowledge of requirements associated with the installation, maintenance, and removal of ESC measures, stabilization, locations of ESC measures and how they are to be maintained, procedures to follow, and when and how to conduct inspections, record applicable findings, and take corrective actions.  The stormwater team shall have ready and easy access to the specified documents and reports.

308.01.08 General Notes.

- Project Information.

  - Total Area Disturbed.				Enter LOD acres

  - Total Cut (Not for bidding purposes).		Enter total cut CY

  - Total Fill (Not for bidding purposes).		Enter total fill CY

  - Offsite Waste/Borrow Area Location (If known). Enter location

  - Stream Use.					Enter stream use

  - Stream Tier (I, II, III).				Enter Tier

  - Stream Impairments.	 			List

- Notifications.

  - Notify the Administration’s Quality Assurance Division (QAD) in writing or by telephone at 410-365-0164 at least 7 working days prior to the Pre-Construction Meeting.

  - Notify QAD at least 7 working days prior to the initial ESC Field Meeting.

  - When the limit of disturbance (LOD) is greater than or equal to 1 acre, notify the pertinent regional office of the Maryland Department of the Environment’s (MDE’s) Compliance Program by telephone 14 calendar days prior to the initial ESC Field Meeting.

    - Central Division (Anne Arundel, Baltimore, Calvert, Charles, Harford, and St. Mary’s counties): 410-537-3510.

    - Western Division (Allegany, Garrett, and Washington counties): 301-689-1480.

    - Western Division Satellite Office (Carrol, Frederick, Howard, Montgomery, and Prince George’s counties): 301-665-2850.

    - Eastern Division (Caroline, Cecil, Dorchester, Kent, Queen Anne’s, Somerset, Talbot, Wicomico, and Worcester counties): 410-901-4020.

  - Notify QAD upon installation of initial ESC measures.

  - Notify QAD prior to installation of sediment traps or sediment basins.

  - Notify QAD prior to removing ESC measures.

  - Notify QAD at Final Acceptance by the Administration.

- Conformance Requirements.

Refer to 308.01.03 and the following.

SWM and ESC approvals will be kept up to date. ESC approval remains valid for 3 years from the date of approval per COMAR 26.17.01.08, after which approvals may be extended or renewed by the Administration’s Plan Review Division (PRD).  Changes to the regulations, terms and conditions of approvals and permits, and the ESC Handbook will be immediately incorporated into the contract by default.  If approvals have lapsed, cease dewatering and other discharges from the project and permanently stabilize disturbed areas.  Do not resume grading or other earth disturbing activities until approvals and permits have been extended or renewed.

- Inspections.

  - Perform daily inspections and inspect as follows.

    - Areas that have been cleared, graded, or excavated and that have not yet completed final permanent stabilization.

    - Installed ESC measures and Stormwater Pollution Prevention Plan (SWPPP) measures.

    - Sediment that has been tracked out from the site onto paved roads, sidewalks, or other paved areas.

    - Material, waste, borrow, and equipment storage.

    - Areas where stormwater typically flows within the project, including drainageways designed to divert, convey, or treat stormwater.

    - Dewatering operations and locations of discharge, including ESC measures that treat the dewatering discharges as well as channelized flows of water to and from those measures.

    - All points of discharge from the project.

    - All locations where stabilization measures have been implemented.

  - Perform inspections within 24 hours following storm events of 0.25 inches or greater rainfall (either the same day the rainfall event concludes or the next calendar day) of installed ESC measures and ensure the measures are always in place and functioning as specified.

  - Maintain ESC measures throughout the duration of the Contract, regardless of whether construction activities are active or inactive.  Maintain access to all ESC measures until they are removed.

  - Perform routine maintenance, repairs, removal and resetting, and replacement as needed and as directed. Take corrective actions following triggering events, severe weather events, and as directed.  Ensure effective operating condition of ESC measures at all times.

  - Maintain a daily log of ESC inspections that includes routine maintenance as well as corrective actions necessitated by triggering events.  Note the rainfall amount when inspections are performed following storm events of 0.25 inches or greater rainfall.

  - Ensure prohibited discharges are not released and preventative measures are in place to prevent prohibited discharges from occurring.

  - When the LOD is greater than or equal to 1 acre, ensure dewatering activities and turbidity monitoring is performed, recorded, documented, and ensure documentation includes corrective actions resulting from triggering events and the date the corrective actions are made.

  - When discharging to Tier II waters, perform inspections of installed ESC measures within 24 hours following storm events of 0.25 inches or greater rainfall and when runoff from snowmelt is sufficient to cause a discharge.  Ensure ESC measures are always in place and functioning as specified.

  - When dewatering turbidity benchmarks apply, inspect discharge points at least once per day on each day discharge occurs.

  - Identify incidents of noncompliance observed and maintenance or corrective actions taken.

  - Identify if the area was subject to flooding.

- Daily Reports.

Complete a daily inspection report that includes the following.

  - The date and time of the inspection.

  - The name(s) of the individual(s) who performed the inspection.

  - Weather information (conditions during the inspection as well as time and amount of last recorded precipitation).

  - A summary of the inspection findings, including maintenance or corrective actions, an assessment of the condition of ESC measures, and how deficiencies were or are being addressed.

  - A description of the present phase of construction.

  - When applicable, the rain gauge or weather station readings that triggered the inspection.

- Additional Recordkeeping.

  - Throughout the duration of the Contract, maintain the following records and ensure they are available for review and inspection by the Administration, MDE, and/or the US EPA, either physically on-site or electronically accessible, or at a readily accessible location within a reasonable distance from the construction activities.

    - The SWM/ESC approval, modification approvals, renewals, and extensions.

    - The Approved ESC Plans, including approved addenda, redlines, and ESC field modifications.

    - The daily reports.

    - When the LOD is greater than or equal to 1 acre, maintain the following additional records.

      - The National Pollutant Discharge Elimination System (NPDES) Notice of Intent (NOI) permit and the NOI application, and associative documents.

      - The Stormwater Pollution Prevention Plan (SWPPP).  MDE may require public access to portions of the SWPP upon request.  If an on-site location is unavailable when no personnel are present, post the notice of the SWPPP location near the main entrance of the construction site.

      - The Approved ESC plans.

      - The Approved SWM plans.

      - A copy of the General Permit for Stormwater Discharge Associated with Construction Activity, General NPDES Permit number MDRC0000, State Discharge Permit number 20CP0000.

      - When located within a Tier II watershed, a copy of the Antidegradation Checklist.

      - All inspection reports and enforcement actions issued from the Administration, MDE, and the US EPA.

- Conditions/Triggering Events.

The following conditions require immediate corrective actions when found.  Notify the REC and the Engineer prior to taking action.

  - Significantly damaged or full replacement of ESC measures with new ESC measures.  (This does not include standard repair, maintenance, and upkeep.)

  - Missing ESC measures, or ESC measures not installed as specified or as directed.

  - Discharges cause an exceedance of applicable water quality standards.

  - A prohibited discharge has occurred.

  - Dewatering discharges exceed the 150 NTU benchmark for the daily maximum of the turbidity monitoring results.

  - Indications of significant amounts of sediment discharge, including but not limited to earth slides or mud flows; concentrated flows of stormwater such as rills, rivulets, or channels that cause erosion when such flows are not filtered, settled, or otherwise treated to remove sediment; turbid flows of stormwater that are not filtered, settled, or otherwise treated to reduce turbidity; deposits of sediment at the project in areas that drain to unprotected stormwater inlets or catch basins that discharge directly to surface waters; deposits of sediment from the project on public or private streets outside of the permitted construction activity; deposits of sediment from the work area onto adjacent property outside of the LOD; or discharges from the work area to municipal conveyances, curbs and gutters, or streams running through or along the work area where visual observations indicate the discharges differ from ambient conditions in terms of turbidity.

- Corrective Actions.

When one or more conditions/triggering events are observed, or if the REC and/or MDE has verified one or more reported conditions/triggering events, perform the following actions as directed and include them in the daily reports.

  - Within one (1) calendar day inspect ESC measures and verify efficient function. Correct deficiencies immediately. Deficiencies include but are not limited to failing to adhere to the latest approved ESC sequence of construction; failure to maintain buffers; grading beyond the LOD; missing ESC measures; and ESC measures improperly installed or are in need of maintenance.

  - Take action to prevent further conditions/triggering events.  Coordinate efforts with the REC and the Engineer.

  - Within three (3) calendar days of a second triggering event and when the LOD is greater than or equal to 1 acre, notify the pertinent regional office of the MDE Compliance Program and the Administration’s QAD.  Inform them of the date of the event and the condition of the ESC measures.

- Sensitive Areas.

With the approval and assistance of the Engineer, coordinate with the appropriate Administration representatives and with the appropriate regulatory agencies to ensure that all permit conditions are met prior to commencing construction activities within specified sensitive areas.  Sensitive areas include but are not limited to floodplains; wetlands; wetland buffers; Chesapeake Bay critical area; forests; tree conservation and protection areas; archeological sites; historic sites; parkland; rare, threatened, or endangered species and critical habitats; open waters; Tier II watersheds; impaired waters; stream protection zones; and stormwater management (SWM) facilities.

Ensure the ESCM monitors all work in sensitive areas and ensure that reasonable care is taken during work in and adjacent to sensitive areas.

Do not place stockpiles or otherwise store materials or equipment within locations of sensitive areas.

- Tier II Watersheds.

For discharges within a drainage area to, or directly into, a tributary that is designated as Tier II, perform the following.

  - Maximize accelerated stabilization techniques.

  - Do not locate stockpiles, debris, equipment, or otherwise store materials within the Stream Protection Zone (SPZ).

- Impaired Waters.

Impaired waters are those identified for one or more specific impairments as listed in Section 303(d) of the Clean Water Act (CWA).

Discharges to impaired waters may not exceed available wasteload allocation for applicable Total Maximum Daily Loads (TMDLs).

For discharges to waters that are impaired for polychlorinated biphenyls (PCBs) and include demolition of any structure with 10,000 square feet of floor space built or renovated before January 1, 1980, implement controls to minimize the exposure to stormwater of PCB-containing building materials, including but not limited to paint, caulk, and pre-1980 fluorescent lighting fixtures.  Ensure disposal of such materials is performed in compliance with applicable State, federal, and local laws.

- Water Quality Standards.

MDE may determine that discharges may cause, have reasonable potential to cause, or contribute to an excursion above any applicable water quality standard, or are causing or contributing to an impairment of a waterbody.  if such a determination is made, MDE may require changes to ESC measures to adequately address, achieve, and document the identified water quality concerns.

Control discharges as necessary and as directed to meet applicable water quality standards.  Prevent the release of floating debris, oil, grease, scum, sludge, and other floating materials that would cause the receiving water(s) to be unsightly; change existing color to produce objectionable color for aesthetic purposes or interfere directly or indirectly with designated uses; or elevate temperature which interfere directly or indirectly with designated uses.

Record verifiable data and information that are representative of ambient conditions and demonstrate that the receiving water is attaining water quality standards in the daily reports.

- Approvals and Permit Coverage.

When inadequate space or locations are provided for construction support activities within the approved LOD, locate these activities at nearby offsite locations after having obtained all necessary permits from the local jurisdiction and property owner permission.  Provide copies of the approvals, permits, and permissions to the REC and the Engineer.

When the LOD is greater than or equal to 1 acre, and the approved LOD specifies locations for use for on-site construction support activities such as on-site concrete or asphalt batch plants, heavy use areas including but not limited to equipment staging and storage, material storage areas, stockpile locations, temporary storage of on-site excavated material until disposal, and on-site borrow pits, do not allow these areas to be used by others not directly associated with the Contract and permanently stabilize these areas when they are no longer needed and prior to Contract closeout.

- Prohibited Discharges.

Prevent release of prohibited discharges including but not limited to the following.

- Wastewater from concrete washouts.

- Wastewater from washouts and cleanout of stucco, paint, form release oils, curing compounds, and other construction materials.

- Fuels, oils, or other pollutants used in vehicle and equipment operation and maintenance.

- Soaps, solvents, or detergents used in external building washdown.

- Toxic or hazardous substances from a spill or other release.

- Water contaminated by toxic or hazardous substances (e.g. from sites managed under Maryland's Voluntary Cleanup Program (VCP) or Land Restoration Program (LRP) or demolition debris contaminated by PCBs, not addressed by a SWPPP.

- Stormwater Pollution Prevention Plan (SWPPP).

When the LOD is greater than or equal to 1 acre, implement the applicable Stormwater Pollution Prevention Plan (SWPPP) to prevent the discharge of pollutants from spilled or leaked materials.

Within seven (7) calendar days of knowledge of the release, provide a description of the release, the circumstances leading to the release, and the date of the release MDE. additional action may be required.

- Perform equipment and vehicle fueling and maintenance in accordance with the following.

  - Eliminate the discharge of spilled or leaked chemicals, including fuels and oils.  Do not perform activities involving chemicals near waters of the state, stormwater inlets or conveyances and provide containment through the use of spill kits, berms, and other suitable techniques.

  - Ensure adequate supplies are available at all times to handle spills, leaks, and disposal of used liquids.

  - Use drip pans and absorbents under or around leaky vehicles.

  - Dispose of or recycle oil and oily wastes in accordance with other federal, State, tribal, or local requirements.

  - Clean up spills and contaminated surfaces immediately by using dry clean up measures.  Do not clean contaminated surfaces by hosing the area down.

  - Eliminate the source of the spill to prevent a discharge or a continuation of an ongoing discharge.

- Do not wash equipment and vehicles on-site.

- Store, handle, and dispose of building products, materials, and wastes, including but not limited to asphalt sealants, copper flashing, roofing materials, adhesives, concrete admixtures, gravel, and mulch, in a way that minimizes exposure to precipitation and minimize the discharge of pollutants from areas in which these items are stored.

- For pesticides, herbicides, and insecticides, refer to Section 701 and the pertinent vegetation establishment specifications regarding the submittal of necessary integrated pest management forms and pertinent landscape reporting forms.

- For diesel fuel, oil, hydraulic fluids, other petroleum products, and other chemicals, store chemicals in water-tight containers, and provide cover to minimize exposure of containers to precipitation and to stormwater or use a similarly effective means that minimize the discharge of pollutants from areas in which these items are stored, including but not limited to the use of spill kits or provide secondary containment.

Clean up spills immediately, using dry clean-up methods where possible, and dispose of used materials at a location that accepts the type of waste to be disposed.  Do not hose down the area to clean surfaces or spills and eliminate the source of the spill to prevent a discharge or a furtherance of an ongoing discharge.

- For hazardous or toxic wastes, including but not limited to paints, caulks, sealants, fluorescent light ballasts, solvents, petroleum-based products, wood preservatives, additives, curing compounds, and acids, separate hazardous or toxic waste from construction and domestic waste and store waste in sealed containers made of suitable materials to prevent leakage and corrosion, and which are labeled in accordance with applicable Resource Conservation and Recovery Act (RCRA) requirements, and all other applicable federal, State, tribal, or local requirements.

Store all outside containers within appropriately sized secondary containment to prevent spills from being discharged or provide a similarly effective means that prevent the discharge of pollutants from areas in which these items are stored, including but not limited to the use of spill kits.

Dispose of hazardous or toxic waste in accordance with the manufacturer’s recommended method of disposal and in compliance with federal, State, tribal, and local requirements.

Clean up spills immediately, using dry clean-up methods, and dispose of used materials properly.  Do not hose down the area to clean surfaces or spills.

Follow all other federal, State, tribal, and local requirements regarding hazardous or toxic waste.

Eliminate the source of the spill to prevent a discharge or a furtherance of an ongoing discharge.

- For construction wastes including but not limited to packaging materials, scrap construction materials, masonry products, timber, pipe, plastics, Styrofoam, concrete, demolition debris and other trash or building materials, provide waste containers of sufficient size and number to contain wastes.  For waste containers that have lids, keep waste container lids closed when not in use and close lids at the end of each workday and during storm events.  For waste containers that do not have lids, provide either cover to minimize exposure of wastes to precipitation or use a similarly effective means that minimizes the discharge of pollutants. refer to TC-6.10.

  - For sanitary waste, position portable toilets so that they are secure and will not be tipped or knocked over and are located away from waters of the state and stormwater inlets and conveyances. Refer to 103.03.05.

  - For washing applicators and containers used for stucco, paint, concrete, form release oils, curing compounds, and other materials, direct wash water into a leak-proof container or leak-proof and lined pit such that overflows do not occur due to inadequate sizing or precipitation.

  - For washout or cleanout wastes, do not dump liquid wastes in storm sewers or Waters of the State.  Remove and dispose of hardened concrete waste consistent with the handling of other construction wastes.

  - Locate washout and cleanout activities as far away as possible from Waters of the State and stormwater inlets or conveyances, and to the extent feasible, designate areas to be used for these activities and conduct such activities only in these areas.

- For the application of fertilizers, perform the following.

  - Refer to 920.03, 701.03, and as specified regarding development of any necessary integrated pest management form and submittal of any necessary landscape reporting form.

  - Apply fertilizer in conformance with Maryland law, Section 701, the Nutrient Management Plan (NMP) developed for the bioretention soil mix (BSM) or topsoil source of supply, and the pertinent vegetation establishment as specified.

  - Do not apply fertilizer before heavy rains or when ground is frozen, or within 10 feet lateral distance from Waters of the State, or within any stormwater conveyance channel.

  - Follow all other federal, State, tribal, and local requirements regarding fertilizer application, including Agriculture Article section 8-803.4.

  - Do not apply to stormwater conveyance channels.

- In the event of a discharge resulting from a spill of hazardous substances or oil from a construction site where the release is an amount equal to or in excess of a reporting quantity established under either 40 CFR Part 110, 40 CFR Part 117, or 40 CFR Part 302, occurring during a 24 hour period, notify the National Response Center (NRC) in accordance with the requirements of 40 CFR Part 110, 40 CFR Part 117, and 40 CFR Part 302; 1-800-424-8802 or 202-267-2675 (in the Washington, DC Metropolitan Area) and notify MDE between 8 am and 5 pm at 410-537-3510 or all other hours at 866-633-4686.

- Clearing and Grubbing.

Unless otherwise specified, the clearing and grubbing unit size is as specified in 101.01.01 (e).  Do not exceed a single 20-acre grading unit.  Once this first unit is half graded and permanent stabilization is established and approved, work may proceed to a second 20-acre grading unit.  No more than 30 acres may be disturbed at any time.

- Stockpiles.

Locate stockpiles outside of protection zones and buffers, and away from stormwater conveyances, storm drain inlets, and areas where stormwater flow is concentrated.

For stockpiles that will be unused for more than seven (7) days, or three (3) days for slopes steeper than 3 horizontal to 1 vertical (3; 1), apply temporary stabilization as specified.

- Ingress / Egress Controls.

Protect all points of construction ingress and egress and minimize tracking of sediment.

  - Restrict construction equipment to designated entrance and exit points.

  - Use appropriate stabilization techniques.

  - Implement additional track-out controls as necessary to ensure that sediment removal occurs prior to construction equipment exit.

When no SCE provided, keep each piece of construction equipment within the LOD until the work is complete and clean treads/tires prior to leaving the LOD.

Where sediment has been tracked-out onto paved roads, sidewalks, or other paved areas outside of the LOD, remove the deposited sediment by the end of the workday in which the track-out occurs.  Remove the track-out by sweeping, shoveling, or vacuuming these surfaces, or by using other similarly effective means of sediment removal.  Do not flush the roadway or sweep tracked-out sediment into stormwater conveyances, storm drain inlets, or Waters of the State.

- Dust Control.

Minimize dust. On areas of exposed soil, minimize the generation of dust through the appropriate application of water or other dust suppression techniques.

- Standard Stabilization Note.

Stabilize exposed areas. Implement and maintain stabilization measures that minimize erosion from exposed areas.

After initial soil disturbance or re-disturbance, apply permanent or temporary stabilization as follows.

  - Within three (3) calendar days as to the surface of all perimeter controls, dikes, swales, ditches, perimeter slopes, and all slopes steeper than 3 horizontal to 1 vertical (3:1).

  - Within seven (7) calendar days as to all other disturbed areas on the project except for those areas under active grading.

Adequate vegetative stabilization requires 95% groundcover.

Same day stabilization only applies to areas that have been stabilized by non-erodible material including but not limited to aggregate, gravel, poured concrete, and asphalt patching.

- Incremental Stabilization.

Refer to the MDE ESC Handbook for the incremental stabilization of cut and fills.

- Erosion and Sediment Control Excavation.

Dispose of material removed from ESC measures at an approved waste site as specified in Section 201.  Materials may be stored for re-use when approved.  Materials stored for re-use may be reused once dried and only when it meets the requirements for embankments or other suitable uses and is approved.

- Sediment Traps and Basins.

  - Situate traps and basins outside of Waters of the State and natural buffer areas.

  - Prevent traps and basins from collecting water from wetlands.

  - Remove accumulated sediment to maintain capacity and perform routine maintenance to ensure the traps and basins remain in effective operating condition.

- Stream Restriction Periods.

All in-stream work is prohibited during stream closure periods, which protect fish and other wildlife dependent on the stream for spawning or migration.

Do not work within streams during the following closure dates, inclusive, for any given year.

  - Use I and I-P: March 1 to June 15.

  - Use II and Use II-P: June 1 to September 30 and December 16 to March 14.

  - Use III and III-P: October 1 to April 30.

  - Use IV and IV-P: March 1 to May 31.

- Dewatering Practices.

Refer to 308.03.31.

- Turbidity Monitoring Benchmark to Protect Water Quality (When the LOD is Greater Than or Equal to 1 Acre).

When discharges from dewatering enter a Tier II stream or waters listed as impaired for sediment or a sediment-related parameter, comply with the benchmark monitoring requirements as specified and document in the daily reports and reporting forms the procedures used in the SWPPP.

The benchmark threshold is not an effluent limitation; it is an indicator that the dewatering controls may not be working to protect water quality.  Investigate and take corrective actions as appropriate through coordination with the REC and the Engineer.

  - Turbidity Monitoring Requirements.

    - Sampling frequency. Collect at least one turbidity sample from the dewatering discharge each day the discharge occurs.

    - Sampling location.  Take samples at all points where dewatering water is discharged.  Take samples after the dewatering water has been passed through ESC measures and prior to its discharge into a receiving water, constructed or natural site drainage feature, or storm drain inlet.

    - Representative samples.  Ensure samples taken are representative of the dewatering discharge for any given day. follow the US EPA “Inspection and Monitoring Guide for Construction Dewatering” (EPA 833-B-22-001).

    - Test methods. Measure samples using a turbidity meter that reports results in Nephelometric Turbidity Units (NTUs) and conforms with an approved method contained in Part 136 of the Federal Regulations (e.g., methods 180.1 and 2130). Use the meter, and conduct a calibration verification prior to each use, consistent with the manufacturer’s instructions.

  - The daily threshold for turbidity is 150 NTU.

  - Compare the daily maximum of the turbidity monitoring results 150 NTU.

    - If the daily maximum of the turbidity monitoring results exceeds 150 NTU, conduct follow-up corrective action and document corrective action taken. coordinate efforts with the REC and the Engineer.

    - Collect and analyze additional samples per day as directed by the REC or the Engineer and include in the daily reports.

    - Discharge locations are independent of each other and subject to the same monitoring, reporting, and benchmark threshold requirements.

  - Reporting and Recordkeeping.

    - Maintain a report of daily turbidity measurements, calibration, test results, and corrective actions taken.

    - Summarize the data using the specified form and submit to the Engineer and REC each day.  The Engineer will submit reports to MDE using the ePermits System (egov.maryland.gov/mde/npdes/account/login) no later than 28days following the end of each monitoring quarter specified.  If there are monitoring days in which there was no dewatering discharge, or if there is a monitoring quarter with no dewatering discharge, indicate this in the turbidity monitoring report.  Keep copies of completed reports on-site for the duration of the Contract.  Retain all such information for a period of at least three (3) years from the date the Contract is closed out.

| MONITORING QUARTERS AND DEADLINES FOR REPORTING TURBIDITY BENCHMARK MONITORING DATA TABLE |  |  |
| --- | --- | --- |
| MONITORING QUARTER | PERIOD | REPORTING DEADLINE |
| 1 | JANUARY 1 TO MARCH 31 | APRIL 28 |
| 2 | APRIL 1 TO JUNE 30 | JULY 28 |
| 3 | JULY 1 TO SEPTEMBER 30 | OCTOBER 28 |
| 4 | OCTOBER 1 TO DECEMBER 31 | JANUARY 28 |

  - Also include the following for dewatering inspections and reporting.

    - Approximate times that the dewatering discharge began and ended each day.

    - If the dewatering discharge is a continuous discharge that continues after normal business hours, indicate that the discharge is continuous.

    - Average gallons per day discharge rate based on pump capacity and throttle setting.

    - Indication of whether or not pollutant discharges were observed at the point of discharge to receiving waters flowing through or immediately adjacent to the project, to constructed or natural site drainage features, or storm drain inlets, including but not limited to sediment plumes, suspended solids, unusual color, presence of odor, decreased clarity, presence of foam, visible sheen on the water surface or visible oily deposits on the bottom or along the stream bank of the receiving water.

    - The date, exact place, and time of sampling or measurements.

    - The name of the individual(s) who performed the sampling or measurements.

    - The date(s) analyses were performed.

    - The name of individual(s) who performed the analyses.

    - The analytical techniques or methods used.

    - The results of analyses.

    - All calibration and maintenance records and all original strip chart recordings for continuous monitoring instrumentation.

- Utility Tie-In Work.

When utility tie-in work is necessary beyond the approved LOD and ESC measures, and the associated cumulative LOD increase for all utility tie-in work is less than 400 sq. ft., perform the following.

  - Call "Miss Utility" at 1-800-257-7777 or 811 at least 48 hours prior to the start of work.

  - Place excavated material on the high side of trenches.

  - Backfill, compact, and permanently stabilize all trenches by the end of each workday for utility tie-in work.

When the associated cumulative LOD increase for all utility tie-in work exceeds 400 sq. ft., do not proceed.  Alert the REC and the Engineer.

- Field Modifications.

Refer to 308.01.06 and 308.03.06.

- Shutdowns / Liquidated Damages.

When the project is found to be in non-compliance or is adversely impacting sensitive areas, Waters of the State, or other protected resources, the Administration will impose partial or total shutdowns and may additionally impose ESC liquidated damages until compliance is restored.  Other penalties may also be incurred.  MDE reserves the right to perform unannounced inspections and impose additional requirements, restrictions, or penalties.

- Design Certification.

ENVIRONMENTAL INFORMATION

PRD # Enter PRD number

DESIGN CERTIFICATION

I HEREBY CERTIFY THAT THIS HAS BEEN DESIGNED IN ACCORDANCE WITH THE ESC HANDBOOK, THE MARYLAND STORMWATER DESIGN MANUAL AND TECHNICAL MEMORANDA, THE ENVIRONMENT ARTICLE SECTIONS 4-101 THROUGH 116 AND SECTIONS 4-201 AND 215, AND THE CODE OF MARYLAND REGULATIONS (COMAR) 26.17.01 AND COMAR 26.17.02 FOR EROSION AND SEDIMENT CONTROL AND STORMWATER MANAGEMENT, RESPECTIVELY.

___________________________________        _________________________________

NAME                                                            SIGNATURE

___________________________________	      _________________________________

MARYLAND REGISTRATION NUMBER                                DATE

P.E., R.L.S. OR R.L.A. (Circle one)

"PROFESSIONAL CERTIFICATION.  I HEREBY CERTIFY THAT THESE DOCUMENTS WERE PREPARED OR APPROVED BY ME, AND THAT I AM A DULY LICENSED PROFESSIONAL ENGINEER UNDER THE LAWS OF THE STATE OF MARYLAND, LICENSE NO.____________________, EXPIRATION DATE:____________________."

308.02 MATERIALS

| Riprap | 901.02.01 |
| --- | --- |
| Stone for Channels and Ditches | 901.03 |
| Stone for Slopes | 901.04 |
| Stone for Gabion | 901.05 |
| 4 in. to 7 in. Stone | 901.05 |
| Asphalt Mixes | Section 904 |
| Pipe | Section 905 |
| Gabion Wire | Section 906 |
| Steel Plate | 909.02 |
| Welding Material | 909.03 |
| Fence Fabric for Super Silt Fence | 914.01.01 |
| Geotextile, Woven and Non-Woven | 919.01, Class E |
| Geotextile, Woven Slit Film | 919.01, Class F |
| Soil Amendments | 920.02 |
| Compost | 920.02.05, Type C |
| Fertilizer | 920.03 |
| Mulch | 920.04 |
| Soil Stabilization Matting | 920.05 |
| Seed and Turfgrass Sod | 920.06 |
| Straw Bales | 921.08 |
| 2 in. to 3 in. Stone | M 43, No. 2 |
| 3/4 in.to 1-1/2 in. Stone | M 43, No. 4 |
| No. 57 Stone | M 43, No. 57 |

308.02.01 Filter Log Casing.  Produced from 5 mils thick continuous high-density polyethylene or polypropylene, woven into a tubular mesh netting material with openings in the knitted mesh of 1/8 in. to 3/8 in.

308.03 CONSTRUCTION

308.03.01 Preliminary Activities.  Prior to beginning construction activities, complete the following.

- Attend the Pre-Construction Meeting and present a general overview of how ESC measures shall be implemented for the Contract.

- Assign an Erosion and Sediment Control Manager (ESCM) and submit to the Engineer the name and credentials of the ESCM for approval.  The ESCM shall hold valid certifications as specified in 308.01.02.

308.03.02 Erosion and Sediment Control (ESC) Preparation.  Prior to establishing staging (heavy use) areas or stockpile areas, and prior to beginning grading and other earth disturbing activities, complete the following.

- Attend the Initial ESC Meeting with the Engineer and the Regional Environmental Coordinator (REC) to discuss, at a minimum, ESC for the project, responsibilities, corrective action procedures, and modification procedures.  When the Contract Documents indicate an LOD greater than or equal to 1 acre, the meeting shall include an invitation to a representative from the Maryland Department of the Environment’s (MDE’s) Compliance Program (MDE Compliance Inspector and the establishment of the Stormwater Team.)

- When the Contract Documents indicate an LOD greater than or equal to 1 acre, post at a safe, publicly accessible location in proximity to the construction area(s), a notice of National Pollutant Discharge Elimination System (NPDES) permit coverage for stormwater discharges associated with construction activities.

Locate the notice such that it is visible from the public right of way that is nearest to the construction activities.  (For linear work zones that extend over several miles and for which a posting may be insufficient to provide notice of permit coverage, submit proposed alternatives of notification to the Engineer, who will coordinate with the REC and the MDE Compliance Inspector for approval.)

The notice shall include the following, be accessible to the public, and be a minimum of 24 in. by 36 in.

  - The Contract Number and project name as listed on the permit.

  - The permittee as SHA.

  - The words “General Permit for Stormwater Associated with Construction Activity” and the permit authorization number.

- Submit, to the Engineer and REC for acceptance and approval, the location of the nearest official National Weather Service (NWS) gauge station or submit a proposal to install a gauge station for the specific location(s) of work.  Proposals shall include detailed information regarding the type, location, accuracy, methodology, and security of the rain gauge.

The rain gauge shall meet NWS large 20 in. gauge standards and have a measuring stick, overflow can, collector funnel, and an inner measuring tube that holds 2 in. of precipitation.  The rain gauge shall use aluminum supports and be installed to where the bottom of the gauge is approximately 10 in. to 15 in. above the ground.

- Demarcate all wetlands, wetland buffers, 100-yr floodplains, waters of the United States (WUS), tree protection areas, stream protection zones (SPZs), special protection areas, and LOD as specified in Section 107.  Ensure the demarcations are inspected and accepted by the Engineer.

308.03.03 ESCM and Superintendent.  Do not commence work without both an approved ESCM and a Superintendent meeting the certification requirements as specified in 308.01.02.

Do not perform excavation, grading, or dewatering activities without the presence of the ESCM.  When excavation, grading, or dewatering activities require an alternate ESCM, submit to the Engineer for approval the name and credentials of an alternate ESCM meeting the certifications as specified in 308.01.02.  Inform the Engineer when the alternate ESCM will be responsible.

If the certifications for the ESCM expire or are revoked, cease construction activities and immediately submit to the Engineer for approval the name and credentials of a new ESCM meeting the certifications as specified in 308.01.02.  Resume construction activities when directed by the Engineer.

The Administration reserves the right to require reassignment of the ESCM duties to another individual at any time with detailed justification.

If the certifications for the Superintendent expire or are revoked, cease construction activities and immediately replace the Superintendent with another meeting the requirements as specified in 308.01.02.  Resume construction activities when directed by the Engineer.

Proactively prevent deficiencies. Promptly resolve deficiencies identified by the Regional Environmental Coordinator (REC) and maintain compliant quality assurance ratings. Refer to 308.03.07.

308.03.04 Off-Site Waste Areas.  Obtain approvals and permits from the appropriate jurisdictional authority for off-site waste areas that are not included in the Contract Documents and coordinate with the Engineer.  Provide a copy of the approvals and permits to the Engineer, who will place in the Contract records.

308.03.05 MDE Inspections.  All areas associated within the Contract are subject to routine, periodic, scheduled, unscheduled, and random field inspections by one or more MDE Compliance Inspectors.  If non-compliance is found, the MDE Compliance Inspector will immediately notify the Engineer and require corrective actions.  The non-compliance and/or corrective actions may require a shutdown of construction activities until the non-compliance is satisfactorily resolved.

308.03.06 Field Modifications.  Alternatives or variations to the ESC Approved Plans, including but not limited to changes to the ESC sequence of construction, location and placement of ESC measures, type of ESC measures, additional or redundant ESC measures, location of staging (heavy use) areas within the LOD, and location of stockpiles within the LOD, may be submitted, at the option of the Contractor, to the Administration for approval.  Such alternatives are known as ESC field modifications.

Submit modification requests to the Administration using the Quality Assurance (QA) Toolkit at least 14 days prior to the desired time to implement the change.  When an ESC field modification request changes the LOD for projects in which the LOD is greater than or equal to 1 acre, submit modification requests at least 30 days prior to the desired time to implement the change.  Requests to change the total LOD for projects in which the total LOD is less than 1 acre but will equal or exceed 1 acre as a result of the ESC field modification are unacceptable.

Submit ESC field modifications overlaid on the pertinent ESC Approved Plans.  Ensure requested ESC field modifications to the ESC Approved Plans are submitted in PDF format, that changes are blue in color, and are overlayed on the pertinent full-size ESC Approved Plans, typically 36 in. x 24 in. in size.  Ensure the ESC field modifications adhere to the standard symbology and names of ESC measures used in the Contract Documents and conform to 308.01.03.  Ensure the request is legible, clear, and easily understood.

The Administration reserves the right to require ESC field modifications to be prepared, stamped, and sealed by a Professional Engineer, licensed in the State of Maryland.

Do not implement ESC field modifications until they have been approved via the QA Toolkit; however, if MDE or another regulatory authority directs ESC field modifications, or if the Engineer determines a modification is needed to prevent discharges, immediately implement the direction and follow up with a submission in the QA Toolkit.

308.03.07 Quality Assurance Ratings.  The REC will routinely inspect the project to ensure compliance with the Contract Documents with regards to ESC and Stormwater Management (SWM).  The REC will assign scores based on these inspections.  The scores will be reported on Form No. OOC61/QA-1:  Erosion/Sediment Control and Stormwater Management Field Investigation Report.

The REC will use the scores to determine the following ratings.

| QUALITY ASSURANCE RATING TABLE |  |
| --- | --- |
| SCORE | RATING |
| >90.0 | A |
| 80.0 - 89.9 | B |
| 70.0 - 79.9 | C |
| 60.0 - 69.9 | D |
| < 60.0 | F |

- Rating A.  A project complies. Corrections to deficiencies may be necessary.

- Rating B.  A project generally complies. Corrections to deficiencies are necessary.

- Rating C.  A project marginally complies.  Corrections to deficiencies are necessary. Shutdown conditions may arise quickly. The REC will reinspect within 72 hours. If the deficiencies have not been satisfactorily corrected, a ‘D’ rating will be issued.  Inspection frequency may increase.

If a consecutive 'C' rating is issued, the REC will reinspect after 72 hours. Failure to achieve a ‘B’ or better rating will result in a ‘D’ rating.

- Rating D.  A project is in non-compliance.  Corrections to deficiencies are necessary. The Administration will shut down earthwork operations and will impose liquidated damages for each day of a 'D' rating. The REC will reinspect within 72 hours.  Inspection frequency may increase.

Failure to achieve a 'B' or better rating will result in an 'F' rating and the entire project will be shutdown.  Liquidated damages will be imposed for each day of a ‘F’ rating.  Inspection frequency may increase.

- Rating F.  A project is in non-compliance.  Corrections to deficiencies are necessary.  An 'F' rating indicates one or more of the following.

- A score of less than 60.

- Required permits and approvals have not been obtained or have expired.

- The approved LOD has been exceeded.

- Sensitive areas as specified have been encroached upon without prior necessary and adequate approval.

- The work is not proceeding in conformance with the Contract Documents.

The Administration will shut down earthwork operations until all deficiencies are satisfactorily corrected and ’B’ or better rating is achieved. The Administration will impose liquidated damages for each day of a 'F' rating.  Inspection frequency may increase.

Following an initial ‘F’ rating, subsequent ‘F’ ratings will result in the following until a ‘B’ rating or better is achieved.

    - The Administration will shut down the entire project.

    - The ESC Training Certificate (Yellow Card) issued by the Administration will be immediately revoked from the Superintendent, and the ESCM responsible at the time of the subsequent rating, for no less than six-months and during which time access to the QA Toolkit will be restricted, after which time the individual may be eligible to take the Administration’s “Erosion and Sediment Control Certification Training for Contractors and Inspectors” (Yellow Card) again. Upon meeting 308.01.02, they may resume their former roles on the project.

    - If degradation to a sensitive area, protected resource, or Waters of the State occurs or has already occurred, or if corrective actions have not been taken, the Administration may elect to have corrective actions performed by another contractor or by Administration personnel.

308.03.08 Severe Weather Event.  Maintain, repair, remove and reset, or replace damaged or dysfunctional ESC measures, and install missing ESC measures, beginning within 24 hours following a severe weather event when safe conditions prevail.

308.03.09 Prohibited Discharges.  Prevent the release of prohibited discharges throughout the duration of the Contract.

308.03.10 Stabilization Requirements.  Following initial soil disturbance, complete temporary or permanent stabilization as specified.

308.03.11 Stabilized Construction Entrance (SCE).  Construct stabilized construction entrances (SCEs) as specified.  Rehabilitate SCEs with periodic top dressing using additional aggregate, replacing the drainage pipe beneath the SCE when one is installed, or making other repairs to the SCE and sediment trapping devices.

When necessary and as directed, place wash racks to prevent tracking of mud and sediment.

308.03.12 Side or Berm Ditches and Culverts.  Construct side ditches in fill areas and berm ditches in cut areas.  Protect linings from sediment deposits.  Place silt fence along the banks of existing streams as specified prior to placing culverts.  To help avoid sedimentation during construction, divert the streams around the location of the culvert until the proposed culvert and channel are stabilized as specified.

308.03.13 Erosion and Sediment Control Original Excavation.  Excavate, construct embankments, grade, and backfill for sediment traps, sediment basins, and other pertinent ESC measures.

308.03.14 Erosion and Sediment Control Cleanout Excavation.  Remove accumulated sediment from ESC measures and other areas during routine inspection and maintenance of ESC measures when necessary and as directed, per the ESC Handbook.

Dispose of removed sediment as excess or unsuitable material.  Alternatively, removed sediment may be reused once it is dried and if it meets embankment requirements unless otherwise specified.

308.03.15 Heavy Use Areas.  Heavy use areas may be used for staging and for storage of equipment and non-erodible materials.  Locate heavy use areas using optional areas that may be identified for use in the Contract Documents.  For locations within the approved LOD, these areas may not be shared with other entities not directly performing work on the Contract.

Alternatively, obtain property owner permission and appropriate regulatory approvals and permits for off-site locations from the appropriate jurisdictional authority(ies) and coordinate with the Engineer.  Provide copies of documented permission, approvals, and permits to the REC and Engineer, who will keep copies of the documents with the Contract records.  Do not allow approvals and permits to expire for the duration of the Contract or as long as the location(s) is (are) needed.

308.03.16 Stockpile Areas.  Stockpile areas may be used for the storage of erodible materials, including but not limited to sand and soils.  Locate stockpile areas using the optional areas that may be identified for use in the Contract Documents.  For locations within the approved LOD, these areas may not be shared with other entities not directly performing work on the Contract.

Alternatively, obtain property owner permission and appropriate regulatory approvals and permits for off-site locations from the appropriate jurisdictional authority(ies) and while coordinating with the Engineer.  Provide copies of documented permission, approvals, and permits to the REC and the Engineer who will keep copies of the documents with the Contract records.  Do not allow approvals and permits to expire for the duration of the Contract or as long as the location(s) is (are) needed.

308.03.17 Onsite Concrete Washout Structures.  Locate onsite concrete washout structures and cleanout activities as far away as practical from sensitive areas, Waters of the State, stormwater inlets, and stormwater conveyances.  Direct wash water into an approved leak-proof container or leak-proof and lined pit designed so that no overflows occur due to inadequate sizing or precipitation.

Do not dump washout or cleanout wastes, solid and liquid, in storm sewers or Waters of the State. Dispose of at an approved facility.

308.03.18 Earth Dike (ED).  Do not use sod as vegetative stabilization unless directed.

308.03.19 Temporary Swale (TS).  Do not use sod as vegetative stabilization unless directed.

308.03.20 Perimeter Dike/Swale.  Do not use sod as vegetative stabilization unless directed.

308.03.21 Pipe Slope Drain.  When slope drain is placed on grade, construct interceptor berms to direct flow into the flared end section.

308.03.22 Riprap Inflow Protection.  Construct as specified in Section 312.

308.03.23 Rock Outlet Protection (ROP).  Construct as specified in Section 312.

308.03.24 Gabion Outlet Protection.  Construct as specified in Section 313.

308.03.25 Gabion Inflow Protection.  Construct as specified in Section 313.

308.03.26 Super Silt Fence (SSF).  Use a 7-gauge top tension wire continuously between posts.

308.03.27 Filter Berms (FB).  Use wood chips and up to 50 percent compost.

308.03.28 Filter Log.  Use compost for filter media.  Drive stakes perpendicular to water flow at a maximum of 8 ft intervals.  Restrict vehicular and construction traffic from crossing filter logs.

Once the drainage areas to filter logs are permanently stabilized with vegetation, with concurrence of the Engineer and the REC, remove stakes.  Filter logs may remain in place and vegetated or removed.  If remaining in place, cut filter log casings open, remove all non-biodegradable material, spread the compost as a soil supplement, and perform the pertinent vegetative establishment.

308.03.29 Temporary Stone Outlet Structure (TSOS).  Permanently stabilize the area immediately after removal of the structure.

308.03.30 Temporary Gabion Outlet Structure (TGOS).  Construct as specified in Section 313.  Grade and stabilize the area beneath the structure, immediately upon removal.

308.03.31 Dewatering.  Dewater only when conditions facilitate successful operations and does not alter stream clarity.  When dewatering discharges flow to Tier II stream or Waters of the State indicated as having an impairment of sediment or a sediment-related parameter, comply with benchmark monitoring for turbidity.

Minimize the discharge of pollutants from dewatering operations as follows.

(a) Route dewatering water through ESC measures designed to minimize discharges of pollutants and prevent discharges from altering visual turbidity or clarity.

(b) Do not discharge visible floating solids or foam.

(c) To the extent feasible, use well-vegetated, upland areas of the site to infiltrate dewatering water before discharge.  Do not use Waters of the State as part of the treatment area.

(d) Use stable, erosion-resistant surfaces, including but not limited to well-vegetated grassy areas, clean filter stone, or geotextile underlayment, to discharge from dewatering controls.

(e) Do not place dewatering operations on steep slopes.

- At all points where dewatering water is discharged, use velocity dissipation measures.

(g) With backwash water, either remove and dispose or return it to the beginning of the treatment process.

(h) For filter bags, determine filter bag dimensions necessary to provide the required storage volume, determine pump and hose sizes, and install.  Do not place filter bags on steep slopes.

(i) For portable sediment tanks, determine the dimensions necessary to provide the necessary storage volume.

308.03.32 Sediment Traps.  Excavate and grade sediment traps to the specified length, width, and depth.

When grading and paving operations are complete and permanent vegetative stabilization within the drainage areas to sediment traps, including slopes and channels, have been applied and those areas are vegetated with at least a 3 inches of growth, with the concurrence of the Engineer and the REC refill sediment traps with suitable materials, shape, and permanently stabilize as specified.

308.03.33 Stone for Sediment Control.  Place No 57 aggregate, 3/4 in. to 1-1/2 in. stone, 2 in. to 3 in. stone, 4 in. to 7 in. stone, and riprap as specified.

308.03.34 Maintenance of Stream Flow.  Maintain the continuous flow of waterways as specified.  Ensure that all excavation performed within the protected stream area is performed in a dewatered condition, which may require additional pumps, sheeting, shoring, cofferdams, and other measures.

With concurrence of the Engineer and REC, adjust stream diversion as necessary to ensure satisfactory performance of the diversion, using additional ESC measures and equipment as appropriate.

Always securely anchor the stream diversion system in place to prevent movement during high water and storm events.  Submit to the Engineer for approval the method of anchoring before implementing.  Do not install anchors beyond the LOD or infringe on the available channel area for stream flow.

Upon completion of construction and when temporary drainage devices are no longer necessary, with the concurrence of the Engineer and the REC, remove the stream diversion.

308.03.35 Removal of ESC Measures.  Do not remove ESC measures until all disturbed areas have been permanently stabilized, vegetation is at least 4 inches tall, exhibits dark green color, and has achieved at least 95 percent groundcover.  Do not remove ESC measures without concurrence from the REC and the Engineer.

308.04 MEASUREMENT AND PAYMENT

The payment will be full compensation for all material, labor, equipment, tools, and incidentals necessary to complete the work.

Replacement of ESC measures that are damaged and replaced as a result of a Severe Weather Event will be measured and paid for at the Contract unit prices provided that a Quality Assurance Rating of “B” or better was obtained in the previous rating. Other damages as a result of an event are subject to TC-7.03.

Maintenance, repairs, rehabilitation, removal, and corrective actions of and to ESC measures will not be measured, but the cost will be incidental to the Contract price for the pertinent item.

The cost of posting a notice of National Pollutant Discharge Elimination System (NPDES) permit coverage for stormwater discharges associated with construction activities will be measured and paid per the temporary sign item.

Corrective actions, due to one or more ‘F’ ratings or are otherwise necessary due to degradation to sensitive areas, protected resources, or Waters of the State due to the Contractor’s action or in-action, directly or indirectly, or performed by Administration personnel or another contractor under the direction of the Administration will be at the Contractor’s expense.

308.04.01 Erosion and Sediment Control Manager ESCM.  The Erosion and Sediment Control Manager (ESCM) will not be measured but the cost will be incidental to the ESC items.  Replacement of the ESCM with a new ESCM will be at no additional cost to the Administration.

308.04.02 Claims.  No claims against the Administration will be considered due to a shutdown of the grading operations or the entire project that resulted from non-compliance.  When corrective actions are performed by another contractor or by the Administration, all costs associated with the work will be billed to the original Contractor.

308.04.03 Incentive Payments and Liquidated Damages Amounts.  Applicable amounts as specified.

308.04.04 Incentive Payments.  When incentive payments are specified payment schedule is as follows, beginning at the Notice to Proceed.

(a) Rating Quarters and Quarterly Incentive Payments.  A rating quarter is a three-month period in which at least 4 inspections are performed by the REC.  An average score of at least 85.0 for the entire quarter is necessary to receive the quarterly incentive amount.  Payment will be made within 60 days from the end of the quarter.  No incentive will be paid for partial quarters or for three-month periods with less than 4 inspections by the REC.  Time extensions approved by the Administration that increase the number of eligible rating quarters will be drawn from the Final Incentive Payment.

(b) Disqualification for Incentive Payments.  Ratings quarters in which a ‘D’ or ‘F’ rating occurs and/or an MDE enforcement action is issued are disqualified from the quarterly incentive for the quarter(s) in which they occurred, regardless of the average score for those entire quarter(s) and no payment will be made.

(c) Final Incentive Payment.  When no ‘D’ or ‘F’ ratings have been received, the average score for the Contract is at least 85.0, and no MDE enforcement actions have been issued, the final incentive payment will be made at Final Closeout in the amount specified or the remaining amount if additional eligible rating quarters were added by time extensions approved by the Administration.

308.04.05 Liquidated Damages.  When 'D' or 'F' ratings are received, liquidated damages will be assessed, and deductions imposed.

308.04.06 Stabilized Construction Entrance (SCE).  Stabilized Construction Entrance (SCE) will be measured and paid for at the Contract unit price per each.

When drainage pipe under the SCE is specified, the pipe will not be measured but the cost will be incidental to the SCE item.

308.04.07 Wash Racks for Stabilized Construction Entrance.  Wash Racks for Stabilized Construction Entrance will be measured and paid for at the Contact unit price per each.

308.04.08 Erosion and Sediment Control Original Excavation.  Erosion and Sediment Control Original Excavation will be measured and paid for at the Contract unit price per cubic yard.

308.04.09 Erosion and Sediment Control Cleanout Excavation.  Erosion and Sediment Control Cleanout Excavation will be measured and paid for at the Contract unit price per cubic yard.

308.04.10 Temporary Mulch.  Temporary Mulch will be measured and paid for as specified in 704.04.01.

308.04.11 Temporary Seed.  Temporary Seed will be measured and paid for as specified in 704.04.02.

308.04.12 Turfgrass Sod.  Turfgrass Sod will be measured and paid for as specified in 708.04.01.

308.04.13 Soil Stabilization Matting (SSM).  Soil Stabilization Matting (SSM) will be measured and paid for as specified in 709.04.

308.04.14 Heavy Use Area Protection.  Heavy use area protection will not be measured but the cost will be incidental to the pertinent ESC measures.

308.04.15 Stockpile Areas.  Stockpile areas will not be measured but the cost will be incidental to the pertinent ESC measures.

308.04.16 Earth Dike (ED).  Earth Dike will be measured and paid for at the Contract unit price per linear foot. 4 Inch to 7 Inch. stone, temporary seeding, and soil stabilization will be measured and paid for as specified in 308.04.60, 704.04, and 709.04, respectively.

308.04.17 Temporary Swale (TS).  Temporary Swale will be measured and paid for at the Contract unit price per linear foot. 4 Inch to 7 Inch stone, temporary seeding, and soil stabilization matting will be measured and paid for as specified in 308.04.60, 704.04, and 709.04, respectively.

308.04.18 Perimeter Dikes/Swales.  Perimeter Dikes/Swales will be measured and paid for at the Contract unit price per linear foot.  Temporary seeding and soil stabilization matting will be measured and paid for as specified in 704.04 and 709.04, respectively.

308.04.19 Temporary Storm Drain Diversion.  Temporary Storm Drain Diversion will be measured and paid for at the Contract unit price per linear foot of the size specified.

308.04.20 Temporary Asphalt Berm.  Temporary Asphalt Berm will be measured and paid for at the Contract unit price per linear foot for 5 Inch Asphalt Berm.

308.04.21 Clear Water Diversion (CWD).  Clear Water Diversion will be measured and paid for at the Contract unit price per linear foot of the size specified.  The payment will include pipe, connections, anchors, sandbags, sheeting, dewatering, and turbidity monitoring when monitoring is required.

308.04.22 Temporary Barrier Diversion.  Temporary Barrier Diversion will be measured and paid for at the Contract unit price per linear foot.  The payment will include barrier, sandbags, sheeting, dewatering, and turbidity monitoring when monitoring is required.

308.04.23 Mountable Berm.  Mountable Berm will be measured and paid for at the Contract unit price per each.

308.04.24 Diversion Fence.  Diversion Fence will be measured and paid for at the Contract unit price per linear foot.

308.04.25 Pipe Slope Drain (PSD).  Pipe Slope Drain (PSD) will be measured and paid for at the Contract unit price per linear foot of the size specified.  The payment will include excavation, backfill, flared end section, geotextile, anchors, coupling bands, and pipe elbows.

308.04.26 Stone Check Dam.  Stone check dam will be measured and paid for at the Contract unit price per ton for the following.  The payment will include geotextile, excavation, and backfill.

    - 4 Inch to 7 Inch Stone for Sediment Control.

    - Washed aggregate as 3/4 Inch to 1-1/2 Inch Stone for Sediment Control.

308.04.27 Riprap Inflow Protection.  Riprap Inflow Protection will be measured and paid for per ton for Riprap for Sediment Control.

308.04.28 Gabion Inflow Protection.  Gabion Inflow Protection will be measured and paid for per cubic yard.

308.04.29 Rock Outlet Protection.  Rock Outlet Protection will be measured and paid for at the Contract unit price per square yard for Riprap Slope and Channel Protection.

308.04.30 Plunge Pool.  Plunge Pool will be measured and paid for at the Contract unit price per square yard for Riprap Slope and Channel Protection.

308.04.31 Silt Fence (SF).  Silt Fence (SF) will be measured and paid for at the Contract unit price per linear foot.

308.04.32 Silt Fence on Pavement.  Silt fence on pavement will be measured and paid for at the Contract unit price per linear foot for Silt Fence.

308.04.33 Super Silt Fence (SSF).  Super Silt Fence (SSF) will be measured and paid for at the Contract unit price per linear foot.

308.04.34 Clear Water Pipe Through Silt Fence or Super Silt Fence.  Clear water pipe through silt fence or super silt fence will not be measured but the cost will be incidental to the pertinent clear water diversion and silt fence items.

308.04.35 Filter Berm.  Filter Berm will be measured and paid for at the Contract unit price per linear foot.

308.04.36 Filter Log.  Filter Log will be measured and paid for at the Contract unit price per linear foot for the size specified.

308.04.37 Temporary Stone Outlet Structure (TSOS).  Temporary Stone Outlet Structure (TSOS) will be measured and paid for per ton for 2 Inch. to 3 Inch Stone for Sediment Control.  The payment will include the baffle board, stakes, and geotextile.

308.04.38 Temporary Gabion Outlet Structure.  Temporary Gabion Outlet Structures will be measured and paid for at the Contract unit price per each.

308.04.39 Standard Inlet Protection.  Standard Inlet Protection will be measured and paid for at the Contract unit price per each for Inlet Protection.

308.04.40 At Grade Inlet Protection.  At grade inlet protection will be measured and paid for the Contract unit price per each for Inlet Protection.

308.04.41 Curb Inlet Protection.  Curb inlet protection will be measured and paid for at the Contract unit per each for Inlet Protection.

308.04.42 Median Inlet Protection.  Median inlet protection will be measured and paid for at the Contract unit per each for Inlet Protection.

308.04.43 Median Sump Inlet Protection.  Median sump inlet protection will be measured and paid for at the Contract unit per each for Inlet Protection.

308.04.44 Combination Inlet Protection.  Combination inlet protection will be measured and paid for at the Contract unit per each for Inlet Protection.

308.04.45 Gabion Inlet Protection.  Gabion inlet protection will be measured and paid for at the Contract unit per each for Inlet Protection.

308.04.46 Catch Basin Insert.  Catch basin insert will be measured and paid for at the Contract unit price per each for Inlet Protection.

308.04.47 Dewatering Practices.  Dewatering practices will be measured and paid for at the Contract unit price for the pertinent dewatering item.  The payment will include turbidity monitoring when monitoring is required.

    - Removable Pumping Station.  Removable Pumping Station will be measured and paid for at the Contract unit price per each.  The payment will include excavation, pipe, geotextile, wire mesh, steel plate, stone, hose, pump, and connections.

    - Sump Pit.  Sump Pit will be measured and paid for at the Contract unit price per each.  The payment will include excavation, pipe, geotextile, wire mesh, steel plate, stone, hose, pump, and connections.

    - Portable Sediment Tank.  Portable Sediment Tank will be measured and paid for at the Contract unit price per each.  The payment will include pipe, geotextile, wire mesh, steel plate, hose, pump, and connections.

    - Filter Bags.  Filter Bags will be measured and paid for at the Contract unit price per each.  The payment will include pump, hoses, connections, mulch, compost, woodchips, sand, and straw bales.

308.04.48 Sediment Traps.  Sediment traps, including ST-I, ST-II, and ST-III, will be measured and paid for at the Contract unit price for one or more of the pertinent items as follows.

    - Erosion and Sediment Control Original Excavation.  Erosion and Sediment Control Original Excavation as specified in 308.04.08.

    - Pipe.  Pipe as specified in 303.04.

    - Stone and Aggregate.  Stone and aggregate will be measured and paid for at the Contract unit price per the pertinent item for the following.  The payment will include geotextile, excavation, and backfill.

      - Riprap for Sediment Control.

      - No. 57 Stone for Sediment Control.

      - 3/4 Inch to 1-1/2 Inch Stone for Sediment Control.

      - 2 Inch to 3 Inch Stone for Sediment Control.

      - 4 Inch to 7 Inch Stone for Sediment Control.

    - Geotextile.  Geotextile will not be measured but the cost will be incidental to the Contact unit price for the pertinent stone and aggregate item(s).

    - Inflow Protection.  As follows.

      - Riprap Inflow Protection.  Riprap Inflow Protection will be measured and paid for per ton for Riprap for Sediment Control.

      - Gabion Inflow Protection.  Gabion Inflow Protection will be measured and paid for per cubic yard.

    - Baffle Board and Stakes.  Baffle board and stakes will not be measured but the cost will be incidental to the Contract unit price for the pertinent inflow protection item(s).

    - Temporary Risers.  Temporary Risers will be measured and paid for at the Contract unit price per each.

    - Anti-Seep Collars.  Anti-Seep Collars will be measured and paid for at the Contract unit price per each.

308.04.49 Sediment Basins.  Sediment basins will be measured and paid for at the Contract unit price for one or more of the pertinent items as follows.

    - Erosion and Sediment Control Original Excavation.  Erosion and Sediment Control Original Excavation as specified in 308.04.08.

    - Pipe.  Pipe as specified in 303.04.

    - Stone and Aggregate.  As specified in 308.04.48(c).

    - Geotextile.  Geotextile will not be measured but the cost will be incidental to the Contract unit price for the pertinent stone and aggregate item(s).

    - Baffle Board and Stakes.  Baffle board and stakes will not be measured but the cost will be incidental to the Contract unit price for the pertinent stone and aggregate item(s).

    - Temporary Risers.  Temporary Risers will be measured and paid for at the Contract unit price per each.  The payment will include trash racks, draw down devices, concrete bases, projection collars, and riser connectors.

    - Modifying Stormwater Management Riser Structures.  Modifying Stormwater Management Riser Structures will be measured and paid for at the Contract unit price per each for Convert Stormwater Management Riser for Sediment Control.  The payment will include installing and removing dewatering pipe systems and restoring stormwater management riser structures to the original condition.

    - Anti-Seep Collars.  Anti-Seep Collars will be measured and paid for at the Contract unit price per each.

308.04.50 Temporary Access Bridge.  Temporary Access Bridge will be measured and paid for at the Contract lump sum price.

308.04.51 Temporary Access Culvert.  Temporary Access Culvert will be measured and paid for at the Contract unit price per linear foot.

308.04.52 Onsite Concrete Washout Structures.  Onsite concrete washout structures will not be measured but the cost will be incidental to the pertinent concrete mixes.  The payment will include implementation of controls associated with pollution prevention activities.

308.04.53 Vegetative Stabilization.  Vegetative stabilization will be measured and paid for at the Contract unit price for the pertinent temporary and permanent item(s).  Reapplication of the pertinent stabilization of permanently stabilized vegetated areas that are disturbed or re-disturbed by grading operations, failure to maintain controls, general negligence, or other activities not specifically authorized by the appropriate approval authority will be at no additional cost to the Administration.

308.04.54 Stone and Aggregate for Sediment Control.  As follows. Geotextile, excavation, and backfill will not be measured but the cost will be incidental to the Contract price.

    - Riprap for Sediment Control.  Riprap for Sediment Control will be measured and paid for at the Contract unit price per ton.

    - Class I Riprap for Slope and Channel Protection.  Class I Riprap for Slope and Channel Protection will be measured and paid for at the Contract unit price per square yard.

    - Class II Riprap for Slope and Channel Protection.  Class II Riprap for Slope and Channel Protection will be measured and paid for at the Contract unit price per square yard.

    - No. 57 Stone for Sediment Control.  No. 57 Stone for Sediment Control will be measured and paid for at the Contract unit price per ton.

    - 3/4 Inch to 1-1/2 Inch Stone for Sediment Control.  3/4 Inch to 1-1/2 Inch Stone for Sediment Control will be measured and paid for at the Contract unit price per ton.

    - 2 Inch to 3 Inch Stone for Sediment Control.  2 Inch to 3 Inch Stone for Sediment Control will be measured and paid for at the Contract unit price per ton.

    - 4 Inch to 7 Inch Stone for Sediment Control.  4 Inch to 7 Inch Stone for Sediment Control will be measured and paid for at the Contract unit price per ton.

308.04.55 Maintenance of Stream Flow.  Maintenance of Stream Flow will not be measured but will be paid for at the Contract lump sum price.  Diversion structures necessary to satisfactorily divert the stream flow, anchoring of the system, excavation, backfill, dewatering the site and excavation within the stream diversion area, sandbags, polyethylene sheeting, diversion pipes, pumps, hoses, connections, portable sediment tanks, maintenance, repairs, rehabilitation, removal, removal and resetting, relocation, corrective actions of and to ESC measures, turbidity monitoring when required, and implementation of controls associated with pollution prevention activities especially with regards to stream clarity, cleanup, and restoration of the stream diversion area that is damaged will not be measured, but the cost will be incidental to the Contract lump sum price.

Payment will not be adjusted for alternative stream diversion systems regardless of changes in quantities from that shown in the Contract Documents.  No payment adjustments will be made for corrective actions to meet stream clarity discharge requirements.

Cleanup and restoration of the stream diversion area that is damaged and replaced as a result of a Severe Weather Event will not be measured but paid for as specified in TC-7.03.

308.04.56 Sandbags.  When sandbags are specified for use other than when incidental to 304.04.22, 304.04.23, 308.04.56, or otherwise specified as incidental, sandbags will be measured and paid for per cubic yard.