CATEGORY 100

PRELIMINARY

Nuclear Density Gauge Storage

DESCRIPTION

Furnish, install, and maintain storage container and hardware exclusively for storing a Nuclear Density Gauge (NDG) assigned to the construction project.

MATERIALS

| Steel Storage Container min. dimensions | 10 ft. x 8 ft. x 8 ft., welded/bolted lock box with extreme security padlock |
| --- | --- |
| Steel Chest (job site box) min. dimensions | L48 in. x H28 in. x W24 in. with enclosed dual lock system, 16-gauge sheet metal body |
| Heavy Duty Steel Chain | Min. 3/16 in. welded link |
| Steel Cable | Min. 3/16 in. diameter |
| Galvanized Steel Bolts | Min. 5/16 in. diameter |

Note: Type A transportation case (provided by MDOT SHA, OMT) approximate dimensions – 30 in. x 17 in. x 14 in.

CONSTRUCTION

Inside Office Storage Requirements:

- Nuclear Density Gauge (NDG) must be stored in compliance with 10 CFR 20 – “Standards for Protection Against Radiation” - https://www.nrc.gov/reading-rm/doc-collections/cfr/part020/index.html.  Recommended safe distance in all directions from the gauge location to any/all permanent workstation(s) is 15 feet.

- The NDG designated storage shall be in a separate lockable room or closet.  The designated storage room/closet shall be without windows and/or doors to the outside.

- This room/closet shall be for the sole purpose of providing an extreme security NDG lockup storage location.

- No other equipment/supplies shall be stored in the NDG security lockup room/closet.

- The locked in its transport case NDG, shall be secured inside a locking steel chest.  The steel chest shall be secured by heavy duty metal locking chain or cable that is bolted, welded, and/or chained inside the container to the wall and/or floor.  The steel chest can be bolted to the floor of the storage container as an alternative to the chain/cable attachment.

- The access door to the designated NDG storage place must be secured by a bolt and locked with an extreme security padlock on the outside of the door.

- Access to the NDG storage place shall be strictly limited to personnel, authorized by the Radiation Safety Officer (RSO), wearing their assigned nuclear radiation monitoring dosimeter badge.

Outside Storage Requirements.

- Nuclear Density Gauge (NDG) must be stored in compliance with 10 CFR 20 – “Standards for Protection Against Radiation”- https://www.nrc.gov/reading-rm/doc-collections/cfr/part020/index.html.  Recommended safe distance in all directions from the gauge location to any/all permanent workstation(s) is 15 feet.

- Separate steel storage container without windows, i.e., ‘C’ Container, Conex Box.

- The separate container is for the sole purpose to provide an extreme security NDG lockup storage location.

- No other equipment/supplies shall be stored in the NDG security lockup container.

- The lock in its transport case NDG, shall be secured inside a locking steel chest.  The steel chest shall be secured by heavy duty locking chain or cable that is bolted, welded, and/or chained inside the container to the wall and/or floor.  The steel chest can be bolted to the floor of the storage container as an alternative to the chain/cable attachment.

- The container door(s) must be secured with an extreme security padlock.

- The extreme security padlock must be located within a security lock housing structure.

- Access to the NDG storage container shall be strictly limited to personnel, authorized by the RSO, wearing their assigned nuclear radiation monitoring dosimeter badge.

Remote Storage Area- additional requirements:

- The r remote storage area of the NDG’s storage container is considered a location away from the MDOT SHA Project Engineer Office and is not under direct daily observation by MDOT SHA project personnel.

- Steel fencing shall be installed around the steel storage container storing the nuclear gauge.

- NDG shall be inspected daily (workdays) by project personnel, authorized by the RSO, wearing their assigned nuclear radiation monitoring dosimeter badge. -. Log of the inspection -shall be kept in the MDOT SHA field office and inside the NDG storage container.

Recommendations:  It is recommended that adequate lighting be provided at an NDG remote storage location at the RSO’s discretion.  This is an additional measure to assure safe and secure NDG storage at project sites and safeguard the stealing of the equipment.

MEASUREMENT AND PAYMENT

Nuclear Density Gauge Storage will not be measured but will be paid for at the Contract lump sum price for the pertinent Nuclear Density Gauge Storage specified.

Payment of 50 Percent of the contract Lump Sum price will be payable once the location has been approved. The remaining 50 percent will be paid in equal amounts on each subsequent

monthly estimate. The number of months used for the prorating will be the number estimated to

complete the work. The final month’s pro-rata amount will not be paid until the location has been restored. The payment will be full compensation for all material, labor, equipment, tools, and incidentals necessary to complete the work of furnishing, installing, and maintaining storage container and hardware exclusively for storing a Nuclear Density Gauge assigned to the construction project.

The construction project storage location and setup must be approved by the Maryland Department of Transportation State Highway Administration Radiation Safety Officer prior to assigning a Nuclear Density Gauge to the project.  Provide a 30-day written notification to the Maryland Department of Transportation State Highway Administration Radiation Safety Officer before relocating the Nuclear Density Gauge storage container.