Category 100

Preliminary

Only used in XB Contracts

SECTION 104.02 — Maintenance of Traffic (MOT)

104.02.04 MEASUREMENT AND PAYMENT

DELETE:  The first paragraph “Unless otherwise specified, Maintenance of Traffic will not … Payment of the Contract lump sum price will be prorated… will be the number estimated to complete the work.” in its entirety.

INSERT:  The following.

Maintenance of Traffic will not be measured but will be paid for at the Contract lump sum per location as described in the Item Description.

The payment will be full compensation for all engineering and work related to the development and application of the TCP(s), as well as all related items of work including furnishing, placement, maintenance, removal, reinstallation, and relocation of devices; and the necessary labor, tools, equipment and incidentals such as lights, Traffic Manager, etc., for which a specific bid item has not been established, to ensure the desired result of a safe and efficient work zone.   Unless an item is found in the schedule of prices, Flagging Operation, Automated Flagger Assistance Devices, Temporary Orange Construction Fence, Cones/Barrels, Barricades, Temporary Barriers and End Treatments, Reflective Markers, Delineators, Temporary Pavement Markings, Arrow Panels, Detours, and Signs for temporary traffic control will be incidental to the Maintenance of Traffic.  When bid items have been established for temporary traffic signs, plastic drums, and traffic cones, these items will only be paid once in the contract.  After initial payment, all use and relocation of these items will be incidental to the Maintenance of Traffic items.

The payment will be full compensation for relocating, turning, completely covering, and uncovering or removing and resetting, maintaining in like new condition, and cleaning existing and temporary traffic signs and other traffic control devices.  The payment will include the inventory of all existing pavement markings and the treatment of any other traffic control device not included in these Specifications but necessary for the fulfillment of the Contract requirements and implementation of the approved Traffic Control Plan, and for all material, labor, equipment, tools, and incidentals necessary to complete the work.

Payment of the Maintenance of Traffic item will be made according to the following payment schedule.

- 50 percent of the Maintenance of Traffic will be paid in the first monthly estimate after the Contractor has started work at a location established in the contract.

- 40 percent of Maintenance of Traffic will be prorated and paid in equal amounts on each of the next four monthly estimates.

- 10 percent or the remaining balance of Maintenance of Traffic will be paid in the month following work being completed at the location established in the contract.