Category 200

Grading

SECTION 201 — Roadway Excavation (Class 1, Class 1-A, Class 2)

201.01 DESCRIPTION

201.01.02 excavation.

dELETE:  List “(c) The removal and disposal…as specified in Section 206” in its entirety.

INSERT:  The following:

(c) The removal and disposal of existing pavement, sidewalks, curb, combination curb and gutter, and Monolithic Median when within the limits of Class 1 or Class 2 excavation. Perform this work as specified in Section 206

201.04 MEASUREMENT AND PAYMENT

delete:  Subsection 201.04.05 in its entirety.

INSERT:  The following.

201.04.05 Removal of existing pavement, sidewalk, paved ditches, curb, or combination curb and gutter, monolithic median, and full depth saw cuts outside the limits of any class of excavation will be measured and paid for as specified in 206.04.02.

delete:  Subsection 201.04.06 in its entirety.

INSERT:  The following.

201.04.06 Full depth saw cuts, removal of existing pavement, sidewalk, paved ditches, curb, and combination curb and gutter, and monolithic median within the limits of any class of excavation will not be measured but the cost will be incidental to the Contract unit price per cubic yard for the pertinent class of excavation in which it occurs.