Category 200

Grading

SECTION 206 — Removal of Existing Pavement, Sidewalk, Paved Ditches, Curb, or Combination Curb and Gutter

Delete:  SECTION 206 in its entirety.

INSERT:  The following.

SECTION 206 — Removal of Existing Pavement, Sidewalk, Paved Ditches, Curb, or Combination Curb and Gutter, and Monolithic Median

206.01 DESCRIPTION

Remove to full depth and dispose of existing pavement, sidewalk, paved ditches, curb, or combination curb and gutter and monolithic median.

206.02 MATERIALS

Not applicable.

206.03 CONSTRUCTION

206.03.01 Full Depth Saw Cut. Saw cut to full depth the existing pavement, sidewalk, paved ditches, curb, or combination curb and gutter along the lines specified or as directed.

206.03.02 Use of Removed Pavement, Sidewalk, Paved Ditches, Curb, or Combination Curb and Gutter and Monolithic Median. When approved, removed materials may be broken and used in the work. Refer to 204.02.01.

206.03.03 Protection of Retained Pavement, Sidewalk, Paved Ditches, Curb, or Combination Curb and Gutter and Monolithic Median. Protect all sections designated to remain from being damaged. Repair or replace damaged areas.

206.04 MEASUREMENT AND PAYMENT

The payment will be full compensation for all material, labor, equipment, tools, and incidentals necessary to complete the work.

206.04.01 Full depth saw cuts, removal and disposal of existing pavement, sidewalk, paved ditches, curb or combination curb and gutter, and monolithic median within the limits of any class of excavation will be incidental to the Contract unit price for the Class of Excavation in which it occurs.

206.04.02 Saw cuts, removal and disposal of existing pavement, sidewalk, paved ditches, curb or combination curb and gutter, and monolithic median outside the limits of any class of excavation will be measured in the original position and paid for as follows:

- Saw Cuts per linear foot when an item is included in the Contract Documents.

- Removal of Existing Pavement, Sidewalk, and Paved Ditches per cubic yard.

- Removal of Existing Curb or Combination Curb and Gutter per linear foot.

(d) Removal of Existing Monolithic Median per square foot.

When any material included in (b), (c), or (d) is removed but not replaced, backfilling with common borrow, subsoil, topsoil, temporary stabilization, and permanent stabilization as directed by the Engineer will not be included in Contract Unit Price.  Payment for this work will be made using the applicable items included in the Contract Documents.