Category 400

Structures

SECTION 403 — Temporary Support of Excavation or Fill

403.03 CONSTRUCTION

INSERT:  The following after the second paragraph.

Do not attach bracing for temporary support of excavation to permanent existing or proposed structures, unless specified in the plans or approved in writing by the Office of Structures.  Requests should include calculations prepared by a professional engineer registered in the State of Maryland, showing that the temporary bracing will not affect the stability, maintenance, and the long term performance of the permanent structure.

Do not extend any temporary support structures directly under any portion of proposed structures and pipes or within 5 ft of proposed pile locations, unless specified in the plans or approved in writing by the Office of Structures.  Contractors should consider the configuration of the batter on piles when installing the supports.