Category 400

Structures

SECTION 420 — Portland Cement Concrete Structures

420.03 CONSTRUCTION

420.03.04 Concreting.

(e) Forming Concrete Parapets and Median Barriers on Bridges

DELETE:  The first paragraph “Either the slip form… when railing is specified.” in its entirety.

INSERT:  The following.

Use the conventional fixed form method for parapets and median barriers, unless special slip forming details for parapets or other slip forming notes are included in the Plans.  When slip forming is allowed, it is the Contractor’s responsibility to determine whether this method of construction will work within the limitations of their own equipment at no additional cost to the Administration.  Do not use the slip form method on bridges maintaining traffic in stages when any portion of the superstructure remains connected to the portion of deck where slip forming will occur or on parapets when railing is specified.

DELETE:  The seventeenth paragraph “Except for the slip form machine… are in progress.”  in its entirety.

INSERT:  The following.

Except for the slip form machine and its concrete supply trucks, do not allow any other construction vehicles on the bridge while slip forming operations are in progress and at least 24 hours after the slip formed concrete has been placed.