Category 400

Structures

SECTION 430 — Metal Structures

- 430.01 DESCRIPTION

- DELETE:  The first paragraph, “Furnish, fabricate, transport...structural steel, respectively.” in its entirety.

- INSERT:  The following.

- Furnish, fabricate, transport, and erect steel beams, plate girders, trusses, grillages, columns and bents, shoes, pedestals, castings, beam retrofit plates and angles, miscellaneous steel, and all incidental structural steel as specified in the Contract Documents or as directed by the Engineer.  Refer to Section 432 for bearings and Sections 435 and 436 for cleaning and painting new and existing structural steel, respectively.

- 430.02 MATERIALS

- ADD:  The following.

| Paint | - Section 435, Section 436, and 912.05 |
| --- | --- |
| Metal Reinforced Epoxy Filler | - 909.12 |

- 430.03 CONSTRUCTION

- ADD:  The following after subsection 430.03.33.

- 430.03.34 Beam Retrofit Operations. As a first order of work, take measurements of the existing retrofit locations.  Alert the Engineer of any major differences between the existing conditions and Contract Plans.  There will be no payment of retrofit elements that do not fit snug against the existing steel.  Provide shop drawings of all elements as specified in 430.03.01.

- Clean all new steel used for steel beam retrofits as specified in 436.03.10(h).  Retrofits which are shop fabricated shall be coated with Paint System B as specified in Section 435.  Retrofits which are field fabricated shall be coated with Paint System C as specified in Section 436.

- If needed, refer to Section 480 for jacking beam operations.

- If there are no other cleaning and painting operations specified in the Contract Documents other than that which is needed to complete the retrofit repairs, the Contractor need not be SSPC-QP1 or SSPC-QP2, Category A certified to perform the field cleaning and painting in the steel repair areas.  If the Contract Documents state that the connection is slip critical, perform the cleaning of the existing steel repair area as specified in 436.03.10(f) or 436.03.10(h) or a combination thereof.  Perform cleaning of all other existing steel repair areas as specified in 436.03.10(d) or 436.03.10(e) or a combination thereof.

- In areas where bolted connections are used, apply metal reinforced epoxy filler to areas of section loss and pitting prior to installing the new steel plates.  Apply the metal reinforced epoxy filler in accordance with the manufacturer’s recommendations and ensure that it remains workable immediately prior to placing the new steel plate. Apply a sufficient quantity of the filler so that excess material will be squeezed out during the installation of the bolts.  Properly dress excess material in a workmanlike manner prior to hardening.

- Unless otherwise specified, complete the last two System B or C coats of paint in the field. Perform all field painting as specified in Section 436.

- 430.04 MEASUREMENT AND PAYMENT

- ADD:  The following after subsection 430.04.04.

430.04.05 The retrofit repairs for the lengths specified in the Plans will be measured and paid for at the Contract unit price per each for the pertinent Structural Steel Beam Retrofit item.  The payment will also be full compensation for the cleaning, painting, and application of metal reinforced epoxy filler.