Category 400

Structures

SECTION 432 — Bearings

432.02 MATERIALS

ADD:  The following to the list of materials.

| Nonshrink Grout | 902.11(c) |
| --- | --- |

432.03 CONSTRUCTION

432.03.02 Installation.

ADD:  The following after the last paragraph “Refer to the...to steel superstructures.”

For bearing retrofit and replacement projects, inspect anchor bolts for deterioration and structural integrity at each existing beam end to be jacked.  Only remove and replace those anchor bolts as indicated in the Contract Documents or designated by the Engineer.  Use anchor bolts of the same diameter and length as the existing or as detailed in the Contract Documents.

Replace existing bearing assemblies in kind by fabricating to the existing dimensions or as detailed in the Contract Documents.

432.03.04 Elastomeric Bearing Pads.

ADD:  The following at the end of the second paragraph “When elastomeric pads...the adhesive sets.”

Set the beam on its bearing before the epoxy adhesive sets.  Ensure that the elastomeric pad does not shift while the beam is being set on its bearing..

432.04 MEASUREMENT AND PAYMENT

ADD:  The following after the first paragraph, “Bearings will not...pertinent items specified.”

432.04.01 All replacement bridge bearing assemblies required for the bridge will be measured and paid for at the Contract unit price per each on the pertinent Remove and Replace Bridge Bearing item.  The payment will be full compensation for the jacking existing beams, removal of the existing bearing, installation, cleaning, and painting of the new bearing, removal of jacking system, and all material, labor, equipment, tools, and incidentals necessary to complete the work.

432.04.02 All replacement anchor bolts required for the bridge will be measured and paid for at the Contract unit price per each for the pertinent Anchor Bolt Replacement item.  The payment will be full compensation for jacking existing beams (if needed), coring the holes, grout, replacement anchor bolts, installation, removing jacking system (if needed), and all material, labor, equipment, tools, and incidentals necessary to complete the work.

432.04.03 Replacement bearing pads will be measured and paid for at the Contract unit price per each on the pertinent Replacement of Elastomeric Bearing Pads item.  The payment will be full compensation for jacking existing beams, removal, furnishing, fabricating, installing, etc., of elastomeric bearing pads including surface preparation, adhesive, cleaning and painting the sole plate, removal of jacking system, and all material, labor, equipment, tools, and incidentals necessary to complete the work.