Category 400

Structures

SECTION 481 — Removing Existing Bridge Scuppers and Downspouts

481.01 DESCRIPTION

Remove existing bridge scuppers and fill downspouts where designated on the Plans.

481.02 MATERIALS

| Nonshrink Grout | 902.11(c) |
| --- | --- |
| PVC Pipe | Section 905 |

481.03 CONSTRUCTION

Removing Existing Scuppers and Filling Downspouts.  Completely remove all scuppers in conjunction with the removal of the existing bridge deck.  Removal of the scuppers includes the grates, scupper basin, associated anchorages, and channels holding the scupper basin in place to the superstructure.

Remove downspouts and piping connected to the removed scuppers in their entirety including all hangers, where designated in the plans.  If the downspouts are designated in the plans to remain in place, fill the downspout piping with nonshrink grout.

481.04 MEASUREMENT AND PAYMENT

The payment will include scaffolding, removal of any debris in the scuppers, furnishing and placing nonshrink grout, and all materials, labor, equipment, tools, and incidentals necessary to complete the work.

481.04.01 Removing existing scuppers and filling the downspout piping with grout will not be measured, but the cost will be incidental to the Removal of Portions of Existing Structure item.