Category 600

Shoulders

SECTION 606 — Permanent Traffic Barrier End Treatments

606.01 DESCRIPTION

ADD:  The following after the first paragraph.

Refer to 605.01.01 and 605.01.02.

606.03 CONSTRUCTION

DELETE:  606.03 CONSTRUCTION in its entirety.

INSERT:  The following.

606.03 CONSTRUCTION

606.03.01 Refer to 605.03.01, 605.03.02, and 605.03.03.  Document each step of the installation, and complete and submit to the Engineer the manufacturer installation checklist for the installed product.

606.03.02 End Treatments

- Buried-in-Backslope End Treatment (Type A). Excavate the slope to install these components.  Bury the ends of the traffic barrier, the end treatment anchorage, and the bottom rail, when required, in a cut slope.  Upon installation, backfill and stabilize the area as specified in 606.03.03 to match the adjacent slope.

For single rail systems, use 6 ft posts throughout the entire end treatment.  For double rail systems, use 8 ft posts, except for the last three posts buried in the cut slope.

Construct the end treatment anchorage according to the Book of Standards.

Install traffic barrier W-beam as specified in 605.03.

- One-Sided End Treatment (Type C). Install in a straight line according to the manufacturer’s recommendations and the Book of Standards.

- Two-Sided End Treatment and Crash Cushion (Types D, E, and J). Install these systems as specified by the manufacturer.  Refer to the manufacturer’s recommendations for installation methods and procedures.

- One-Sided Downstream End Treatment (Type K). Install according to the Book of Standards.

- Traffic Barrier W-Beam Radius Anchorage (Type L). Install according to the Book of Standards.

- All Other Traffic Barrier End Treatments. Install these systems as specified by the manufacturer.  Refer to the manufacturer’s recommendations for installation methods and procedures.

- Nose Section. Reflectorize as shown in the Contract Documents and as approved by the Office of Traffic and Safety.

- Permanent Crash Cushion Sand Filled Plastic Barrels (SFPB). Provide the components and assemble, place in the required configuration, and fill each barrel according to the manufacturer's recommendations or as specified in the Contract Documents.  Ensure that each SFPB is watertight and separated from other SFPB by a distance of 3 in.  Place the last row of SFPB 12 in. from the shielded object.

Reflectorize the first barrel of the SFPB configuration as specified.  Mix approved antifreeze agent into loose, dry sand according to the manufacturer’s recommendations and install sand mixture in barrels.

606.03.03 Grading Adjustment. When grading adjustment is required for installation of traffic barrier end treatments and crash cushions, adjust the grading using topsoil, crusher run aggregate CR-6, asphalt millings or grindings, subsoil, common borrow, or other approved material as specified in 605.03.07(a), 605.03.07(b), and 605.03.07(c).

606.04 MEASUREMENT AND PAYMENT

ADD:  The following after Sub-section 606.04.10.

606.04.11 Refer to 605.04.19.

606.04.12 The manufacturer installation checklist signed by the Traffic Barrier W-Beam Manager including their “SHA Traffic Barrier Certification” credential will be required for payment of each end treatment.