Category 600

Shoulders

SECTION 613 — Driveways and Entrances

613.03 CONSTRUCTION

613.03.03 Grading Adjustment.

DELETE: 613.03.03 Grading Adjustment in its entirety.

INSERT: The following.

613.03.03 Grading Adjustment. When grading adjustment is required for installation of driveways and entrances, adjust the grading using topsoil, crusher run aggregate CR-6, asphalt millings or grindings, subsoil, common borrow, or other approved material, as specified in 605.03.07(a), 605.03.07(b), and 605.03.07(c).