Category 900

MATERIALS

SECTION 950 — Traffic Materials

950.12 LUMINAIRES AND LAMPS

950.12.01 Luminaire Construction.

ADD:  The following after the last sentence of the first paragraph in (d).

All LED Roadway Luminaires shall be cobrahead style unless otherwise specified in the Contract Documents.

DELETE: The last sentence of the second paragraph in (d) “The Correlated Color Temperature (CCT) shall be less than 4500 K…greater than 65.”

INSERT: The following as the last sentence of the second paragraph in (d).

The Correlated Color Temperature (CCT) shall be between 3000 K and 4000 K and the Color Rendering Index (CRI) shall be greater than 65.

DELETE: The fourth paragraph in (d).

INSERT: The following as the fourth paragraph in (d).

Provide LED Roadway Luminaire drivers that are Solid State (electronic) type with an input voltage range from 120-277VAC (±10 percent), input frequency of 60Hz, minimum power factor of 90 percent at full load, Total Harmonic distortion less than 20 percent, case temperature rated for -40 C to 50 C, and contain 3 kV input high voltage surge protection.