Category 100

PRELIMINARY

SECTION 104.08 — TEMPORARY TRAFFIC SIGNS (TTS)

DELETE:  SECTION 104.08 — TEMPORARY TRAFFIC SIGNS (TTS) in its entirety.

INSERT:  The following.

SECTION 104.08 — Temporary Traffic Signs (TTS)

104.08.01 DESCRIPTION

Furnish, install, and maintain TTS on or along all transportation facilities.

104.08.02 MATERIALS

| Composite Aluminum Signs, Plastic Signs, Flexible Roll Up Signs, Portable Sign Supports, and Mounting Hardware | QPL |
| --- | --- |
| Sign Material | 950.08 |
| Square Perforated Tubular Steel Sign Supports | As Approved by the Office of Traffic and Safety |
| Reflectorization | 950.03 |
| Wood Sign Supports | 921.05 and 921.06 |

Use fluorescent orange for all orange colored work zone signs. Use reflectorized signs for both daytime and nighttime use.

Temporary traffic signs and supports shall conform to MASH 2016 criteria for Test Level 3.

104.08.02.01 Sign Thickness.

- Aluminum Signs.  Use the following minimum thickness for fabricated aluminum signs mounted on wood or steel tubular posts.

| LONGEST DIMENSION OF SIGN (in.) | MINIMUM THICKNESS (in.) |
| --- | --- |
| ≤ 12 | 0.040 |
| 12+ to 24 | 0.063 |
| 24+ to 36 | 0.080 |
| 36+ to 48 | 0.10 |
| > 48 | 0.125 |

(b) Composite aluminum and flexible Roll Up Signs.  When composite aluminum, plastic, or flexible roll up signs are used on portable supports, the support shall be approved by the Office of Traffic and Safety to hold that sign material.  When supported on portable sign supports, composite aluminum signs shall be at least 0.08 in. thick.

104.08.03 CONSTRUCTION

104.08.03.01 Sign Design and Fabrication.  Design and fabricate signs according to the Administration’s Standard Highway Sign Book (SHSB).  You may request a copy of the SHSB  from the Office of Traffic and Safety, Traffic Engineering Design Division.  Submit designs of signs not included in the SHSB in scaled sketches for approval.  Direct written requests to the Engineer.

All work area warning signs with the exception of the “2(3) LEFT(RIGHT) LANE CLOSED XX” (W20-5a(1)) signs shall be 48 in. x 48 in., unless otherwise specified.  Sign W20-5a(1) shall be 60 in. x 60 in., unless otherwise specified.

104.08.03.02 Sign Installation.  Place signs in accordance with the MdMUTCD and the Contract Documents.  Adjust signs along the highway to provide adequate sight distance to work zone signs and existing signs.  Ensure that signs are not obscured or obstructed.  Ensure that supplemental signs do not cover any part of the face of the primary sign.

Mount signs that will be in place for more than three working days on wood posts or on square perforated tubular steel posts, unless otherwise specified.  Do not place any additional bracing on wood posts unless the sign is behind a protective barrier.  The tops of the wood posts shall not protrude more than 3 in. beyond the nearest edge of the sign.

Portable sign supports shall be self-erecting.  They shall be able to withstand a wind velocity of 70 mph and be able to maintain themselves within 5 degrees of rotation around their vertical axis.

Do not install signs before they are inspected and approved.  Do not display signs to traffic until directed.  Properly maintain the TTS, leave in place while applicable, and remove immediately when no longer required.  When operations are performed in phases or stages, display only those signs to traffic that apply to the present conditions.

When one or more signs do not indicate of actual conditions, such as during periods of temporary shutdown or extended periods of no work being performed (including lunchtimes and overnight periods), remove the entire work zone setup, remove the inapplicable signs, turn them away so they are not visible to any traffic, or completely cover them with an approved opaque material.

When signs are no longer needed, remove temporary traffic signs, associated materials and incidentals from the project site.  Remove materials become the property of the Contractor.

104.08.03.03 Sign Maintenance and Sign Replacement.  Maintain signs in a new or like new condition. Maintain sign faces free of tape, tape residue, or any other foreign matter.  Remove all advertisements from signs and supports.

When directed, replace signs with poor reflectivity or those that become faded, illegible, or damaged.  Signs that are not new may be used only if the reflective intensity at a divergence angle of 0.2 degrees and incidence angle of minus 4 degrees conforms to at least 70 percent of the values as specified in 950.03 over 90 percent of their reflectorized surface.  At other times throughout the duration of the Contract, the sign reflectivity intensity shall be at least 60 percent.  The acceptability of the signs shall be determined by 1 ft square test plates calibrated in accordance with these requirements.

104.08.04 MEASUREMENT AND PAYMENT

104.08.04.01 Temporary Traffic Signs will be measured and paid for at the Contract unit price per square foot of sign for the Temporary Traffic Sign item.  The payment will be full compensation for furnishing the signs and supports, installation, relocation, maintenance, cleaning, replacement due to non-traffic damage and normal wear, removal, and for all materials, labor, equipment, tools, and incidentals necessary to complete the work.

104.08.04.02 Where signs have been set and damaged by traffic, and the Engineer determines that they are not repairable, replacement will be measured and paid for at the Contract unit price per square foot of sign for the Temporary Traffic Sign item.  The payment will be full compensation for furnishing the signs and supports, installation, relocation, maintenance, cleaning, removal, and for all material, labor, equipment, tools, and incidentals necessary to complete the work.