Category 100

PRELIMINARY

SECTION 104.28 — Speed Display Trailer (SDT)

DELETE:  SECTION 104.28 — SPEED DISPLAY TRAILER (SDT) in its entirety.

INSERT:  The following.

SECTION 104.28 — Speed Display Trailer (SDT)

104.28.01 DESCRIPTION

Furnish, install, operate, maintain, relocate, and remove a portable, self-contained, trailer mounted, speed display trailer (SDT) with static speed limit sign.  The SDT includes a static speed limit sign and a speed feedback sign (dynamic radar speed display).

The SDT shall be compliant with the Maryland Manual on Uniform Traffic Control Devices (MdMUTCD) and Maryland Standard Sign Book.

104.28.02 MATERIALS

| Speed Display Trailer | QPL |
| --- | --- |
| Reflectorization | 950.03 |
| Sign Material | 950.08 |

Each SDT shall include the trailer, structural support system, sign panel assembly, lift mechanism, power supplies, trailer mounted controller, non-invasive speed detection device, and ancillary equipment.  All materials for the SDT shall be like new, corrosion resistant, and unaffected by water spray, salt, oil, gasoline, and all other contaminants in the quantities typically found along the edge of a traveled roadway.  Construction, materials, and operation shall be in accordance with NFPA, UL, and NEC.

The SDT trailer shall be painted orange and shall comply with Maryland Motor Vehicle Law.

104.28.02.01 Equipment.

- Speed Feedback Sign (SFS).  A SFS shall supplement the static speed limit sign and shall be positioned below the speed limit sign.  It shall consist of a static and a dynamic display.  The SFS shall not flash, strobe, change color, or use other animated elements integrated into the dynamic display.

- Static.  The static portion of the SFS shall include the YOUR SPEED legend and shall have a black legend on a fluorescent yellow background.  The minimum letter height shall be 6 in. with Series C Standard Alphabet font from the Maryland Standard Sign Book.  Reflective sheeting shall be as specified in 950.03.  The sign shall be rectangular in shape.

- Dynamic.  The format of the message to be displayed on the changeable message sign (CMS) portion of the sign shall be XX where XX is the measured vehicle speed in miles per hour (mph).  The color of the changeable message legend shall be a yellow (amber) luminous legend on a black opaque background.  The minimum LED number height shall be 14 in.  The width of the numbers shall be similar to Series E Standard Alphabet font from the Maryland Standard Sign Book.  The CMS shall produce brightness greater than 10,000 candela per square meter at maximum intensity.  The housing for the dynamic components shall be a minimum National Electrical Manufacturers Association (NEMA) 3R rated.  The display shall adjust automatically for ambient or operational light conditions.  The CMS display may use a full or discrete matrix display.  If discrete matrix display is used, each character shall be displayed using a minimum array of 5 columns by 7 rows with at least two array modules per line.  If a full matrix display is used, the sign shall have a pixel arrangement of at least 5 columns by 7 rows per character with at least 3 LEDs per pixel.  The CMS shall conform to ITE specifications for yellow (amber) color warning applications.  The CMS shall provide full illumination within at least a 24 degree cone perpendicular to the sign face.  The operating temperature range of the LED shall be -30 F to 125 F.

- Static Speed Limit Sign Panel.  A R2-1 (48 in. x 60 in.) Speed Limit sign shall be attached to the trailer.  The sign shall be easily removed and replaced.  The static sign face shall be reflectorized as specified in 950.03.  The top of the Speed Limit sign shall include a Work Zone plaque with a minimum size of 48 in. x 12 in.  The Work Zone plaque shall have a black legend and border on a fluorescent orange background with 6 in. Series C Standard Alphabet font from the Maryland Standard Sign Book.

- Trailer.  In accordance with Maryland Motor Vehicle Law governing trailers.

- Lift Mechanism.

- The lift mechanism shall be installed such that the sign panel can be raised and lowered.

- A corrosion resistant safety bolt or pin shall be provided to prevent the sign panel from lowering once in the raised position.  A self-locking mechanism shall be incorporated into the safety bolt or pin to prevent it from being inadvertently dislodged.

- Electrical Connections and Gauges.

- All wiring from power sources to SDT equipment shall use locking cable connectors.

- Volt and amp gauges shall be provided for DC.

- Standard negative ground system shall be tied to the sign chassis.

- Power Supply.  The SDT shall operate from a solar powered electrical system consisting of battery power and solar array panels.  The electrical system shall be capable of supporting continuous operation of the system for 21 consecutive days without an auxiliary charge, and be capable of being charged by a generator or other commercially available power source in case of malfunction, damage, or failure of the primary solar powered electrical system.

- Controller Requirements.  The controller shall be capable of the following.

- Controlling the SFS operating over a -30 F to 125 F range and in a 20 percent to 95 percent noncondensing humidity range.

- Designed for fail-safe prevention of improper information display in the case of a system malfunction.

- Monitoring and displaying the battery output voltage and solar array actives (charging and discharging).

- Monitoring and displaying the status of the photocell and adjusting the sign illumination to match the ambient light conditions.  The controller shall have at least nine levels of dimming from 10 percent to 100 percent brightness.

- Being contained in a sheet metal or high-density polyethylene (HDPE), weatherproof cabinet located on the controller housing and insulated to protect against excessive vibration and temperature.  The cabinet shall be equipped with a lockable door with hasp/lock setup installed in a manner to maximize its effectiveness in stopping unauthorized access to the controls.  Other physical locking devices may be approved by the Engineer.  The cabinet shall have an interior cabinet light or backlit controller display.

- Having remote access to modify speed thresholds, and download vehicle speed readings.

- Storing vehicle speed readings with a time and date stamp and have sufficient memory to store 100,000 readings.

- Speed Display Software.

- The SDT shall be supplied with a non-invasive speed detection unit.  The unit shall be able to be rotated on a separate vertical axis from the SDT to allow for better aiming and detection.  The unit shall detect and display the speed of only approaching traffic.

- The speed display software shall be able to display vehicle speeds that are traveling over a set threshold.  The SDT shall also be capable of not displaying a vehicle speed over a separate set threshold.

- The SDT speed display software shall have the capability of modifying speed thresholds.

- Warning Beacons.  Warning lights and/or beacons shall not be attached to the sign or the trailer.

104.28.03 CONSTRUCTION

Furnish, install and test the SDT on the project site 24 hours in advance of actual use to ensure that the unit is functioning properly.  Coordinate location with Administration and its Automated Speed Enforcement Contractor for Automated Speed Enforcement.

Place the SDT unit as shown in the Contract Documents or as directed and aim SDT at approaching traffic to ensure sign panel is visible for a minimum distance of 600 ft for nighttime conditions and 800 ft for normal daylight conditions from any point along the travel approach roadway.  Ensure the SDT is level and that the sign face is not obscured by highway alignment or glare from vehicle headlights.  Install temporary concrete traffic barrier or drums around the SDT according to the Book of Standards and the Contract Documents.

The SDT shall be able to withstand wind of 80 mph.

When deployed, the mounting height to the bottom of the speed limit sign shall be a minimum of 7 ft from near edge of traveled roadway in accordance with the MdMUTCD.  The mounting height to the bottom of the SFS shall be high enough to ensure traffic barrier, devices, or other obstructions will not block visibility of the SFS.

The Contractor is responsible for continuous operation of the SDT including any maintenance, repair, troubleshooting, or replacement.

104.28.04 MEASUREMENT AND PAYMENT

104.28.04.01 Speed Display Trailer will be measured and paid for at the Contract price per unit day for each Speed Display Trailer unit.  Each Speed Display Trailer will be paid for only once per unit day, regardless of how many times it is relocated.  When a Speed Display Trailer is used for part of a day, it will be measured and paid for as a unit day.  When operations require continuous 24 hour use of a Speed Display Trailer, the unit day will be measured as 24 hour periods from start to finish.

The payment will be full compensation for the Speed Display Trailer and for all material, labor, equipment, tools, and incidentals necessary to furnish, install, operate, and maintain the Speed Display Trailer as specified or as directed.

104.28.04.02 Signs attached to the Speed Display Trailer will not be measured but the cost will be incidental to the Contract unit price for the pertinent Speed Display Trailer item.

104.28.04.03 Drums for Maintenance of Traffic.  Drums will be measured and paid for as specified in 104.12.

104.28.04.04 Temporary Concrete Traffic Barrier and Reset Temporary Concrete Traffic Barrier shall be measured and paid for as specified in 104.04.04.  Reflective Barrier Markers and Vertical Panels shall be measured and paid for as specified in 104.04.04.

104.28.04.05 Temporary Traffic Barrier End Treatments, Remove and Reset Temporary Traffic Barrier End Treatments and Repairing Temporary Traffic Barrier End Treatments will be measured and paid for as specified in 104.09.04