CATEGORY 400

Structures

SECTION 413 — Micropiles

413.03 CONSTRUCTION

413.03.01(d) Micropile Installation Procedure Submittal.

DELETE:  413.03.01(d)(8)(i) in its entirety.

INSERT:  The following.

- Load test procedures and acceptability criteria for the verification and proof test micropiles, including creep test criteria.  Minimum strength acceptance criterion for verification load tests shall be at 2.0 times the design load test loading.  Minimum strength acceptance criterion for proof load tests shall be at 1.6 times the design load test loading.  The creep test shall hold 1.33 times the design load for at least 1 hour. For the creep test, the maximum criteria for movement shall be 0.04 inches after the first 10 minutes and an additional maximum of 0.08 inches over the last 50 minutes.

413.03.08 Pile Load Tests.

DELETE:  The first paragraph of 413.03.08(a) Verification Load Test in its entirety.

INSERT:  The following.

- Verification Load Tests.  Verification load tests shall be performed on one non-production test micropile per substructure unit, installed near each substructure unit but outside of the footing using method specified in ASTM D3689 (Tension Testing).  If there is no location shown on the plan for this test, the Engineer shall find a suitable location within the limit of disturbance that is between 10 and 20 feet away from the footprint of each substructure unit.  The verification test micropile shall be loaded to the verification test load established in 413.03.01(d).  Do not load the verification test micropile to more than 80 percent of its structural capacity.  The verification test micropile’s structural capacity shall be designed by the micropile design engineer and shall not be less than the proof test and production micropile’s structural capacity shown in the Plans.  Any required increase in strength of the verification test micropile elements above the strength required for the production micropile elements shall be provided for in the Contractor’s bid price.  The verification test shall be performed prior to installation of the proof load test micropiles and any other production micropile.