As-Built Inspection Tabulations/Checklist for BMP Number:

PR No.:	SHA Contract No.:

Accepted by SHA-PRD:

Name		Date

M-8 BIO-SWALES CONSTRUCTION CHECKLIST

| ACTIVITY | ADDITIONAL REQUIRED SUPPORTING INFORMATION | ONSITE INSPECTION DATE | AB INSPECTOR INITIALS | AB ACCEPTANCE DATE |
| --- | --- | --- | --- | --- |
| PRIOR TO EXCAVATION, VERIFIED E&S CONTROLS ARE INSTALLED AND CONFIRMED SURROUNDING AREA IS STABILIZED | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED EXCAVATION ACTIVITES AND VERIFIED STAKEOUTS ARE ACCURATE AND RE-STAKED AS NEEDED | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF GEOTEXTILE AND VERIFIED PROPER INSTALLATION TECHNIQUES WERE PERFORMED AS SPECIFIED | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF NO. 57 AGGREGATE BEDDING FOR SUBDRAIN AND VERIFIED SPECIFIED THICKNESS | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF SUBDRAIN | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF NO. 57 AGGREGATE AROUND AND ABOVE SUBDRAIN AND VERIFIED SPECIFIED THICKNESS | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF NO. 7 AGGREGATE AND VERIFIED SPECIFIED THICKNESS | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF COARSE SAND AND VERIFIED SPECIFIED THICKNESS | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF APPROVED BSM AND VERIFIED SPECIFIED THICKNESS | PHOTOGRAPHS MATERIAL APPROVAL FORM(S) OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION OF CHECK DAMS | PHOTOGRAPHS OTHER (LIST): |  |  |  |
| OBSERVED INSTALLATION AND ESTABLISHMENT OF VEGETATION AND VERIFIED TYPES, MATTING AND QUANTITES SPECIFIED | INSPECTION REPORT PHOTOGRAPHS OTHER (LIST): |  |  |  |

AB Inspector is required to perform inspection on-site for these activities as required by COMAR 26.17.02.10.

Indicate all the dates of inspection for each activity and indicate final acceptance date for the activity in the last column.

Note to Designer: It may be necessary to add or delete features as appropriate for the specific facility.  Notify SHA when these standard tabulations are modified

As-Built Inspection Tabulations/Checklist for BMP Number:

PR No.:	SHA Contract No.:

Accepted by SHA-PRD:

Name		Date

M-8 BIO-SWALES TABULATION

| FEATURE | DESIGN | AS-BUILT | DIFFERENCE | AB INSPECTOR INITIALS | AB INSPECTOR ACCEPTANCE DATE |
| --- | --- | --- | --- | --- | --- |
| BOTTOM WIDTH (FT.) |  |  |  |  |  |
| TOTAL LENGTH (FT.) |  |  |  |  |  |
| MAXIMUM CHANNEL SLOPE (FT./FT.) |  |  |  |  |  |
| LEFT SIDE SLOPE (H:V.) |  |  |  |  |  |
| RIGHT SIDE SLOPE (H:V.) |  |  |  |  |  |
| THICKNESS OF NO. 57 AGGREGATE (IN.) |  |  |  |  |  |
| THICKNESS OF NO. 7 AGGREGATE (IN.) |  |  |  |  |  |
| THICKNESS OF COARSE SAND (IN.) |  |  |  |  |  |
| THICKNESS OF BSM (IN.) |  |  |  |  |  |
| SUBDRAIN PIPE DIAMETER (IN.) |  |  |  |  |  |
| SUBDRAIN PIPE MATERIAL |  |  |  |  |  |
| NUMBER OF CHECK DAMS |  |  |  |  |  |
| DISTANCE BETWEEN CHECK DAMS (See profile) |  |  |  |  |  |

AB Inspector is required to perform inspection on-site for these activities as required by COMAR 26.17.02.10.

Indicate all the dates of inspection for each activity and indicate final acceptance date for the activity in the last column.

Note to Designer: It may be necessary to add or delete features as appropriate for the specific facility.  Notify SHA when these standard tabulations are modified