Office of Materials Technology

Aggregate Bulletin

Test Data

Soils and Aggregate Technology Division

7450 Traffic Drive

Hanover, Maryland 21076

# Aggregate Bulletin Contacts

Amanuel Welderufael, Assistant Division Chief - Lab

443-572-5276 or AWelderufael@mdot.maryland.gov

Alan A. Amador, Aggregate Lab Team Leader

443-572-5275 or AAmador@mdot.maryland.gov

http://www.roads.maryland.gov/OMT/AggBlt.pdf

Toll Free:  866-926-8501

# Maryland State Highway Administration

# Aggregate Bulletin

# Introduction

This bulletin contains the qualified list of aggregate producers with the physical test data. The bulletin will be updated on a quarterly basis. The last day of January, April, July and October of each year will be the publishing date. Published bulletin data will be valid for one year.  For instance, if a test is required to be performed each year, for test data published in the 1st quarter, new test data will be due by the beginning of the 1st quarter of the next year.  We encourage producers to keep track of their records, including due dates and submit their requests for aggregate testing on time.  Producers are responsible for informing the Soils and Aggregate Technology Division (SATD) of any changes to producer information, such as producer name, quarry name, subsidiary producer’s name, contact information, qualified use of aggregate.

The data listed in the aggregate bulletin are considered to be representative of material supplied by a particular source of aggregate. The materials contained within this document are not meant to represent all aggregate materials a particular quarry can produce or supply. It is emphasized that the listed data must not be interpreted in absolute terms.  The information is provided as a guide for the expected quality of the material furnished by a listed source.

# Process for Listing as a Qualified Aggregate Source in MDOT SHA Aggregate Bulletin

- Written Request

The aggregate producer shall submit a written request to:

Amanuel Welderufael, Assistant Division Chief - Lab

Soils and Aggregate Technology

Maryland State Highway Administration

7450 Traffic Drive

Hanover, MD 21076

AWelderufael@mdot.maryland.gov

- The SATD requires the producers to submit their written requests before the quarterly aggregate bulletin update.  Please refer to the flow chart in Section II for “MDOT SHA Annual Quality Testing Process.” If samples are tested at MDOT SHA lab, we are requesting the producers to deliver aggregate samples to the MDOT SHA laboratory 45 days prior to the next publishing date of the aggregate bulletin in order to publish the test data on time.

- Please refer to Appendix A for “Aggregate Quality Test Request” Form. Producers shall complete and submit this form for each material (i.e. No. 57, Concrete Sand, Mortar Sand, etc.) for each quarry.

- Safety Data Sheet (SDS): All requests must be accompanied by a Safety Data Sheet with specific quarry location with the effective date.  No aggregate will be accepted prior to receipt of the SDS.

- MDOT SHA Annual Quality Testing Process

Producer submits request to MDOT SHA (Note 1)

MDOT SHA responds to Producer within 10 working days with required tests.

MDOT SHA Lab

Please Refer to Next Page

Producer responds to MDOT SHA within 10 working days with testing facility. State, Independent Lab, producer (AASHTO accredited)

Independent Lab/Producer Lab

MDOT SHA Lab

Producer prepares sample per AASHTO  R90 and Submits to MDOT SHA Lab/Independent Lab/Producer Lab

Independent Lab/ Producer Lab/ Producer Lab – Retain a split sample for MDOT SHA

Independent Lab – Submits test results and all worksheets/supporting documentation to producer/ MDOT SHA

Results are published in the next Quarterly Aggregate Bulletin update.

SHA reviews, evaluates and approves test results.

Note 1: Aggregate Bulletin will be published quarterly.

Producer Responds to MDOT SHA within 10 working days with Testing Facility

Independent Lab / Producer Lab

MDOT SHA LAB

Is MDOT SHA Contract # Valid?

No

No

Stop and Notify Producer

AASHTO Accredited Lab

Direct Payment?

Yes

Yes

Arrow down

Yes

Test Cost sent to Producer for Payment

Arrow down

Notify Producer to Proceed with Independent Lab Testing and Store Split Sample

Yes

Notify Producer to Deliver Sample to MDOT SHA

Payment Receipt?

No

Please Refer to Previous Page

Please Refer to Previous Page

Pending

# Aggregate Tests, Test Frequency and Sample Quantity Requirements

| Aggregate Category | Use | Required Tests | Specifications | Test Frequency[1] | Material Quantity/Type |
| --- | --- | --- | --- | --- | --- |
| Coarse | General Use (HMA Base, GAB, Backfill, etc.) | Specific Gravity & Absorption | AASHTO T-85 | 1 year | 3 bags of No. 57 (at most 35 lbs. per bag) |
|  |  | Soundness | AASHTO T-104 | 3 years |  |
|  |  | Los Angeles Abrasion (LA) | AASHTO T-96 | 2 years |  |
|  | PCC | In addition to tests for General Use |  |  |  |
|  |  | Alkali-Silica Reaction (ASR) | ASTM C1260 | 3 years |  |
|  | HMA Surface | In addition to tests for General Use |  |  |  |
|  |  | Dynamic Friction Test | MSMT 215 & 216[2] | 2 years | 1 bag of 35 lbs. material passing ½” sieve and retained on 3/8” sieve |
|  |  | In addition to tests for General Use and HMA Surface |  |  |  |
|  | High Friction Materials | Petrography for non-carbonate aggregate | TBD | 2 years | As required per Independent Lab |
|  |  | Acid Insoluble Residue (AIR) for carbonate aggregate | TBD | 2 years |  |
| Fine | PCC (Concrete Sand) | Specific Gravity and Absorption | AASHTO T-84 | 1 year | 2 bags (at most 35 lbs. per bag) |
|  |  | Soundness | AASHTO T-104 | 3 years |  |
|  |  | Alkali-Silica Reactivity (ASR) | ASTM C1260 | 3 years |  |
|  | Mortar Sand and No. 10 Dust | Specific Gravity and Absorption | AASHTO T-84 | 1 year | 2 bags (at most 35 lbs. per bag) |
|  |  | Soundness | AASHTO T-104 | 3 years |  |
| Note: [1] Except for new quarry requesting first time [2] Dynamic friction testing will be performed by MDOT SHA lab only. |  |  |  |  |  |

- Please note that, for quality assurance purposes, SATD has the right to request aggregate tests to be performed more frequently (i.e., Quality Assurance (QA) Testing) than the testing frequency specified above.

- If aggregate samples are tested at the MDOT SHA laboratory, upon the receipt of payment or confirmation of valid contract charge number, the SATD will send a notice to the producer for sampling.  Also, SATD will provide “Aggregate Test for Quality,” Form 12, Appendix B, to producers for logging each sample for each quarry.  The producers are required to complete this form and submit back to the MDOT SHA lab along with samples at the address mentioned above.  The sampling shall be in conformity with AASHTO R90.

- All information provided by a Producer, Fabricator or Supplier will be kept confidential by SHA and OMT.  An exception could occur when the Producer, Fabricator or Supplier makes it publicly available or when agreed upon with SHA and OMT.  If the need arises for the release of confidential information by either law or contractual requirements, the Producer, Fabricator or Supplier would be informed of the release of information unless prohibited by law.  This confidentiality includes information obtained from sources other than the Producer, Fabricator or Supplier (e.g., complainant, regulators).

- Samples submitted to SATD, have to labeled properly with the following information.

Example

- Organization’s Name 		Stone Materials

- Quarry Name	 		Hanover Quarry

- Contract/ Charge No.		AA123B21

- Aggregate Size			#57, #67, GAB, Concrete Sand…etc.

- Date Sampled			01/01/2022

- Test Purpose 			GAB-QA, Aggregate Bulletin…etc.

Map of Quarry Locations

# Appendix A

Aggregate Quality Test Request Form

Office of Materials Technology

# Soils and Aggregate Technology Division

# Aggregate Quality Test Request Form

# Aggregate Producer’s Data Sheet

# Date Submitted:  _____________

# Organization’s Name (Parent Aggregate Producer):

________________________________________________________________________

# Office / Facility’s Name (Subsidiary Aggregate Producer):   Same as above ____ or

________________________________________________________________________

Supervising Office’s Mailing Address:

# Street or P.O. Box:	_________________________________________

# Town / City:	_______________________________________________		State:		__________________	 Zip:	_________________

# E-Mail Address:	_______________________________________________

# Phone No(s):		_______________________   _______________________

# Fax No.:		_______________________

# Quarry Name’s (If Applicable):	___________________________________

# Street or P.O. Box:	_________________________________________

# Town / City:	__________________________ State: _______ Zip:  _________

Entitled Point of Contact:	_________________________________________

# Title / Position:		_________________________________________

# E-Mail Address:	_______________________________________________

# Phone No(s):		_______________________     ______________________

# Fax No.:	____________________

# SDS Provided: Yes _____ No____________

# Testing Charges:  Bill Direct (Yes or No) ______ If No,

# MDOT SHA Contract #/Charge #:______________________

# MDOT SHA Project # (if available):_____________________

# Please provide the following information if applicable:

# Testing Facility: MDOT SHA Lab ______ Independent Lab ____If yes, Lab Name_______

Text Box

# Aggregate Category (ies) for Testing: Coarse______Fine______Other_____  _______________________________________________________________________

# Aggregate Type (s) for Testing: #57______Concrete Sand ______Mortar Sand_____

# #10 Dust_________Other________ _______________________________________________________________________

# Aggregate Use(s): HMA Surface____ HMA Base _____PCC_____GAB _____Other_____General_____

Note:  Shaded areas will be published in the Aggregate Bulletin.

Appendix B

Aggregate Test for Quality

(Form 12)

Testing Completed – Reporting Results.

Upon request, the producer will be notified of the preliminary test results once the testing is completed.  Subsequently, the test results will be entered into SATD’s aggregate database for the next quarterly Aggregate Bulletin update.

# Appendix C - Procedures of Quality Aggregate Testing by Independent Laboratories

# Submittal Requirements to Independent Laboratory

The submission package by each producer shall contain the following:

- Aggregate Quality Test Request Form (Appendix A)

- Safety Data Sheet (SDS) for each test material per each quarry location

- Independent laboratory package including name, address, a list of aggregate tests the lab is qualified to perform, experience, AASHTO accreditation and AMRL Proficiency sample ratings for the last two years.

- The independent lab organization chart and technician’s qualification including point of contact for manager, resumes, training records and experience.

The submittal package shall be provided to SATD via mail or electronically to the following address 45 days prior to the Quarterly Aggregate Bulletin update otherwise the SATD does not guarantee the test data to be included to the next published Aggregate Bulletin:

Attn: Amanuel Welderufael, Assist. Division Chief - Lab

Soils and Aggregate Technology

Maryland State Highway Administration

Office of Materials Technology

7450 Traffic Drive

Hanover, MD 21076

AWelderufael@mdot.maryland.gov

After reviewing the submittals, we will inform you whether the independent lab is accepted or not. If the lab is accepted, the independent lab shall be valid for one (1) year from the date of the acceptance letter.

# Aggregate Tests for Independent Laboratory

The accepted independent lab shall comply with the following aggregate test procedures and specification for aggregate testing.

- AASHTO T11 - Materials Finer Than 75-μm (No. 200) Sieve by Washing

- AASHTO T27 - Sieve Analysis of Fine and Coarse Aggregate

- AASHTO T84 - Specific Gravity (Relative Density) and Absorption of Fine Aggregate

- AASHTO T85 - Specific Gravity (Relative Density) and Absorption of Coarse Aggregate

- AASHTO T96 – Resistance to Degradation of Small-Size Coarse Aggregate by Abrasion and Impact in the Los Angeles Machine

- AASHTO T104 - Soundness of Aggregate by Use of Sodium Sulfate or Magnesium Sulfate

- AASHTO R76 – Reducing Samples of Aggregate to Testing Size

- AASHTO T255 – Total Evaporable Moisture Content of Aggregate by Drying

- ASTM C 1260 - Accelerated Detection of Potentially Deleterious Expansion of Mortar Bars due to Alkali-Silica Reaction Aggregate or Aggregate/Pozzolan Combinations

- AASHTO accredited laboratories shall meet R-18 requirements and participate in the AMRL proficiency testing program. In addition, the laboratories should possess or have access to the equipment, facilities, personnel, and calibrated instruments, which are necessary to test material.

- The producers are encouraged to minimize the number of laboratories used per quarry to minimize errors of precision due to technicians and laboratories to ensure the test quality.

- The producers shall be submitting the independent lab submittal package to MDOT SHA rather than the independent lab submitting their information directly to us.

# Split Samples

The producers shall have a split sample for each quarry before submitting samples to the independent lab.

The SATD will require the split samples as needed to ensure the test quality. The SATD Quality Assurance team may witness and pick the split sample during their normal plant visit. The aggregate producers are required to pay for sampling and reviewing test results in case they do not provide an approved/active contract number and a project charge number.

# Test Results Submission by the Independent Laboratory

The independent lab shall submit the final test results along with raw data to the MDOT SHA lab. The raw data sheet shall include the laboratory letterhead with the technician’s name, test date, address, contact information, and manager signature.  The MDOT SHA will review and verify the test data before publishing in the Aggregate Bulletin.

# Aggregate Bulletin

# Keys

( - )		Under test/under evaluation

( # )	In accordance with the Standard Provision Insert (SPI) of Standard Specifications for Construction and Materials, July 2008, Table 901 D, and Aggregate Physical Property Requirements for Asphalt Mixes (Note (e)):  when carbonate rock is used, it shall have a minimum of 25 percent insoluble residue retained on the No. 200 sieve. Otherwise, the aggregate source does not qualify for use as a high friction aggregate.

(**)	ASR determined in accordance with ASTM C 1293, Determination of length change of concrete due to Alkali Silica Reaction.

DFV 	Dynamic Friction Value is defined as a coefficient of friction multiplied by 100. Accordingly, coarse aggregate is categorized as three broad categories: high, standard, and low DFVs, designated as HDFV, SDFV, and LDFV. These categories are presented below in detail:

HDFV:

- Category I	(HDFV I): DFV is equal to 50

- Category II	(HDFV II): DFV is equal to 45

- Category III	(HDFV III): DFV is equal to 40

SDFV:

- Category IV	(SDFV IV):  DFV is equal to 30

- Category V	(SDFV V): DFV is equal to 25

LDFV:

- Category VI	(LDFV VI): DFV is less than 25