| # Recommend Approval: _____________________________________ Team Leader Date _____________________________________ Division Chief Date | Maryland Department of Transportation State Highway Administration Office of Materials Technology MARYLAND STANDARD METHOD OF TESTS |  |
| --- | --- | --- |
| Approved: _____________________________________ Director Date | PROCEDURES FOR DETERMINING THE RIDE SPECIFICATION PAY LIMITS AND PAY ADJUSTMENTS | ### MSMT 736 |

SCOPE:

These procedures are used to determine the Ride Specification Pay Limits and Pay Adjustments.  The Administration will perform the pay adjustment calculations for the Overall IRI and for Defect Cost and then will calculate the Total Pay Adjustment.

REFERENCE DOCUMENTS:

## Section 535

## MSMT 563

Spreadsheet application for calculating Ride Specification Limits

DEFINITIONS:

The pay adjustment will be calculated based on the pay limits.

Where:

|  | DESCRIPTION | UNITS |
| --- | --- | --- |
| PF | Pay Factor for Overall IRI | Dollars per lane-mile |
| INCENTIVE | Incentive for Overall IRI | Dollars |
| DISINCENTIVE | Disincentive for Overall IRI | Dollars |
| Pdefect,i | Defect Cost for a given 25’ section | Dollars per section |
| DEFECT COST | Sum of the defect costs (Pdefect,i) for all defect sections | Dollars |
| IRIAVG | Overall average IRI for the project | Inches per mile |
| IRIdefect,i | IRI for a given 25’ defect section | Inches per mile |
| NS | Number of tested 25 foot Sections | Sections |

### PROCEDURE:

### 1. Determine The IRI Values for Maximum Incentive, Full Pay, Disincentive And Defects:

Use the spreadsheet application located at:

https://www.roads.maryland.gov/mdotsha/pages/Index.aspx?PageId=32

to calculate IRIa, IRIb, IRIc, IRId and IRIe.

Locate the “RideSpecLimits” tab and use it to calculate IRIa, IRIb, IRIc, IRId and IRIe.  Enter the information into the yellow highlighted cells in Step 1 to compute the pay limits.

2. Compute the Defect Cost:

Use the “RideSpecLimits” tab to compute the defect cost.  Enter the information into the yellow highlighted cells in Step 2 to compute the defect cost for a given defect section.

The remaining spreadsheet tabs show the range of defect costs for each functional class.

Compute the defect cost for each defect section using the appropriate formula for the functional class:

Interstates & Freeways/Expressways:

Pdefect, i = 100 +

All Other Routes:

Pdefect, i  = 100 +

Locals:

Pdefect, i = 80 +

Where:	Pdefect, i		=	Defect cost for section i

IRI defect	=	IRI value of section i

IRIe			=	IRIe obtained from Step 1

The total defect cost for the project is the sum of the defect costs of all 25 ft sections.

Note:  OMT’s Asphalt Technology Division (ATD) computes the total defect cost for every project using the Ridetool 2013 program and in accordance with these procedures.

3. Determine the Maximum Incentive And Disincentive Costs:

Based on the predominant functional classification for the project limits, obtain the maximum incentive (Pmax) and maximum disincentive (Pmin) from the table below:

| FUNCTIONAL CLASS | MAXIMUM INCENTIVE FOR OVERALL RIDE (Pmax in dollars per lane mile) | MAXIMUM DISINCENTIVE FOR OVERALL RIDE (Pmin in dollars per lane mile) |
| --- | --- | --- |
| Interstates & Freeways/Expressways | 7350 | 7350 |
| All other routes | 6150 | 6150 |
| Locals | 5300 | 5300 |

4. Pay Adjustment Calculations:

Overall IRI.  The overall average IRI for the project (IRIAVG) will be calculated as the average IRI value of all tested 25 ft sections on the project.  The pay adjustment for Overall IRI will then be calculated based on the factors below.

Incentive.	PF =		Pmax, when IRIAVG is less than or equal to IRIa

PF =	Pmax x (IRIb - IRIAVG)/(IRIb - IRIa), when IRIAVG is greater than IRIa
		and less than IRIb

INCENTIVE = PF x NS x (25/5280 lane miles per section)

DISINCENTIVE = 0

Full Pay.	When IRIAVG is greater than or equal to IRIb and less than or equal to IRIc

INCENTIVE = 0

DISINCENTIVE = 0

Disincentive.	PF =	Pmin x (IRIAVG - IRIc)/(IRId - IRIc), when IRIAVG is greater than IRIc
		and less than IRId,

PF =	Pmin, when IRIAVG is greater than or equal to IRId

INCENTIVE = 0

DISINCENTIVE = PF x NS x (25/5280 lane miles per section)

Defect Cost.  The IRI for each individual section will be used to calculate any cost to be applied for defects on the project.  This pay adjustment applies only to the pavement within the tested sections.  DEFECT COST = Sum of the defect costs (Pdefect,i) for all defect sections.

Total Pay Adjustment.  A total pay adjustment (TPA) will be made based on the total of any incentive and disincentive for Overall IRI minus any defects.  TPA resulting in increased payment will be paid under the item Pavement Surface Profile Pay Adjustment.  This item amount has been established by the Administration and shall not be revised.  TPA resulting in decreased payment will be deducted from monies owed.  The TPA shall be subject to conditions (a) and (b) below.

Total Pay Adjustment = INCENTIVE - DISINCENTIVE - DEFECT COST

(a)	Regardless of the measured profile of any test section, incentive payment will not be permitted for the project when the Contractor’s QC data was not submitted on time in conformance with 535.03.02.

Total Pay Adjustment = 0 - DISINCENTIVE - DEFECT COST

(b)	The total value of Overall IRI disincentive and Defect Cost shall not be more than the Maximum Disincentive pay adjustment for all of the profiled 25 foot sections.

If DISINCENTIVE + DEFECT COST is greater than Pmin x NS x (25/5280 lane miles per section) then Total Pay Adjustment = - Pmin x NS x (25/5280 lane miles per section)

CALCULATIONS:

Example:

Roadway Classification: Urban Freeway/Expressway

Existing IRI = 225

Scope of paving: Grinding, 0.75” Wedge/Level, and 1.25” HMA overlay

### 1. Determine The IRI Values for Maximum Incentive, Full Pay, Disincentive And Defects:

Based on this information, make the following selections in Step 1 to calculate IRIa, IRIb, IRIc, IRId and IRIe.

Calculate the following values for IRIa, IRIb, IRIc, IRId and IRIe:

2. Compute the Defect Cost:

Based on the IRIe value obtained in Step 1, assume that the following 25 ft sections are classified as “defects”:

Given that the functional classification of the roadway is Urban Freeway/Expressway, use the following formula to compute the defect cost for each defect section:

Pdefect, i = 100 +

Calculate the defect costs for each defect section as follows:

| Begin Station | End Station | L_IRI | R_IRI | Average IRI (IRI defect) | Defect Cost (Pdefect, i) |
| --- | --- | --- | --- | --- | --- |
| 100 | 125 | 185 | 190 | 188 | $ 107.02 |
| 135 | 160 | 212 | 220 | 216 | $ 124.89 |
| 180 | 205 | 175 | 182 | 179 | $ 101.28 |
|  |  |  |  | Total | $ 333.19 |

The total defect cost for the project is the sum of the defect cost of all sections; which equals $333.19

3. Determine the Maximum Incentive And Disincentive Costs:

Given that the roadway functional class is Urban Freeway/Expressway, the maximum incentive (Pmax) for the project is $7,350 per lane mile, and the maximum disincentive (Pmin) is $7,350 per lane mile.